Report on

BAU and Policy Scenarios Demand Forecast – Delhi

Prof. Dr. Sanjay Gupta

Principal Investigator

Pintu Saini

Research Associate

School of Planning and  Architecture, New Delhi

August- 2017

Contents

BAU and Policy scenario Demand Forecast – Delhi	1

1.	BAU Scenario	6

1.1	Background	6

1.2	Horizon Year Road Network	6

1.2.1	New committed road network projects in Delhi	7

1.3	Horizon year Metro system network	7

1.3.1	Proposed Metro Network in Delhi (Phase - III)	7

1.3.2	Proposed Metro Network in Delhi (Phase - IV)	8

1.4	Network Speed Assumptions	9

1.5	Planning Forecast	10

1.5.1	Population Projections	10

1.5.1.2 Planning Zone Level Forecast	12

1.6	Transport demand forecast	19

1.6.2	Trip Distribution	20

1.6.3	Modal Split	20

1.6.4	Trip Assignment	21

2.	Policy Bundles	23

2.1	Police Bundle 1: Planning	23

2.1.1	Increase in Transit Network	23

2.1.2	Promote Transit Oriented Development	28

2.1.3	Combination of Increase in Transit + Transit Oriented Development scenarios	32

2.2	Police Bundle 2: Travel Demand Management	34

2.2.1	Increase in Fuel Cost	34

2.2.2	Congestion Pricing	39

2.2.3	Combination of Fuel pricing + Congestion pricing scenarios	42

2.2.4	Combination of Car Pooling and Car Sharing + Fuel Price + Congestion Pricing	44

2.3	Police Bundle 3: Technology adaptation	47

2.3.1	Electric Mobility	47

2.3.2	BAU scenario with Electric Mobility	48

2.3.3	Police Bundle 1: Planning with Electric Mobility	49

2.3.4	Police Bundle 2: Travel Demand Management with Electric Mobility	50

3.	Comparison of BAU & Policy Scenario	51

3.1	Comparison of BAU & Policy scenario (Transport parameters)	51

3.2	Comparison of BAU & Policy scenario (GHG Emissions)	53

List of Tables

Table 1-1: Existing Link Capacities and Speeds for different category of roads	6

Table 1-2: Additional Metro Lines in Phase - III	6

Table 1-3: Additional Metro Lines in Phase - IV	7

Table 1-4: Existing Link Capacities and Speeds for different category of roads	8

Table 1-5: Comparison of population growth in Delhi with similar metropolitan cities in India	9

Table 1-6: Estimated Population by various methods	10

Table 1-7: Population Estimations by Population Reference Bureau report 2007	11

Table 1-8: Population and Area under various planning zones	12

Table 1-9: Proposed and Revised Holding Capacity in Planning Zone for the year 2021 (‘000)	13

Table 1-10: Population of Planning Zones from 1991 to 2011………………………………………………14

Table 1-11: Population Estimation at Planning Zonal level for Horizon Year 2030 and 2050	14

Table1-12: Proposed residential density in each planning zone a for horizon year 2030 and 2050	15

Table 1-13: Work force participation in Delhi (in Lakhs)	16

Table 1-14: Estimated workforce participation rate for the year 2030 and 2050	17

Table 1-15: Estimated Workers for the Horizon Year 2030 and 2050	17

Table 1-16: Employment estimations at planning zone level for the horizon year 2030 and 2050	18

Table 1-17: Estimated Motorized PCTR for the cardinal year details	19

Table 1-18: Comparison of Average trip length (ATL) in cardinal years	19

Table 1-19: Comparison of mode split in Base year and BAU-2030 & 2050	19

Table 1-20: Comparison of mode split in Base year and BAU-2030 & 2050	19

Table 1-21: Comparison of Base year and BAU-2030 & 2050 scenario	22

Table 2-1: Comparison of Base year, BAU & Transit + TOD - 2030 & 2050	24

Table 2-2: Increased Fuel cost assumptions	25

Table 2-3: Increased fuel cost of various available modes for 2030 & 2050	27

Table 2-4 Comparison of mode split in BAU and Increase in Fuel cost -2030 & 2050	29

Table 2-5: Comparison of mode split in BAU and Increase in Fuel cost -2030 & 2050	29

Table 2-6: Comparative evaluation  of Base year and BAU & Increase in Fuel cost - 2030 & 2050 scenario	31

Table 2-7 Comparison of mode split in BAU & Congestion Pricing - 2030 & 2050	31

Table 2-8: Comparison of mode split in BAU & Congestion Pricing - 2030 & 2050	32

Table 2-9: Comparison of Base year and BAU & Congestion pricing - 2030 & 2050 scenario	33

Table 2-10 Comparison of mode split in Base year and BAU and Fuel pricing + Congestion Pricing -2030 & 2050	35

Table 2-11: Comparison of mode split in Base year and BAU & Fuel pricing + congestion pricing -2030 & 2050	36

Table 2-12: Comparison of Base year and BAU and Fuel pricing + Congestion pricing - 2030 & 2050 scenario	36

Table 2-13 Comparison of mode split of Car sharing + Fuel cost + Congestion pricing -2030 & 2050	36

Table 2-14: Comparison of Base year and BAU & Car sharing + Fuel cost + Congestion pricing -2030 & 2050	38

Table 2-15 : Forecast of Electric Mobility in India - 2030	39

Table 2-16 : Adopted Electric mobility share in Delhi by 2030 & 2050	39

Table 2-17 : BAU with Electric Mobility - Vehicle travelled KM by 2030 & 2050	41

Table 2-18 : BAU with Electric Mobility –Emissions by 2030	41

Table 2-19 : BAU with Electric Mobility –Emissions by 2050	42

Table 2-20 : Police Bundle: Planning with Electric Mobility - Vehicle travelled KM by 2030 & 2050	42

Table 2-21 : Police Bundle: Planning with Electric Mobility - Vehicle travelled KM by 2030	44

Table 2-22 : Police Bundle: Planning with Electric Mobility - Vehicle travelled KM by 2050	45

Table 2-23 : Police Bundle: Travel Demand Management with Electric Mobility - Vehicle travelled KM by 2030 & 2050	46

Table 2-24 : Police Bundle: Travel Demand Management with Electric Mobility - Vehicle travelled KM by 2030	47

Table 2-25 : Police Bundle: Travel Demand Management with Electric Mobility - Vehicle travelled KM by 2050	47

Table 2-26 : BAU with Electric Mobility –Emissions by 2030……………………………………………….. 48

Table 2-27 : BAU with Electric Mobility –Emissions by 2050…………………………………………………48

Table 2-28 : Police Bundle: Planning with Electric Mobility - Vehicle travelled KM by 2030 & 2050……..49

Table 2-29 : Police Bundle: Planning with Electric Mobility -  Emissions by 2030………………………...49

Table 2-30 : Police Bundle: Planning with Electric Mobility – Emissions by 2050…………………………49

Table 2-31 : Police Bundle: Travel Demand Management with Electric Mobility - Vehicle travelled KM by 2030 & 2050………………………………………………………………………………………………………...50

Table 2-32 : Police Bundle: Travel Demand Management with Electric Mobility – emissions by 2030…..50

Table 2-33 : Police Bundle: Travel Demand Management with Electric Mobility – emissions by 2050…50

Table 3-1 : Comparison of BAU & Policy scenario (Transport parameters) – 2030 & 2050	55

Table 3.2 Comparative Evaluation of BAU and Alternate policy scenarios………………………………..…51

List of Figures

Figure 1-1-1: Horizon year Road Network for 2030 & 2050	6

Figure 1-1-2: Horizon year Metro Network for 2030 & 2050	9

Figure 1-1-3: Map of Planning Zone in Delhi	13

Fig. 1-1-4: Population Growth Trends and Projections in Delhi……………………………………………….16

Fig. 1-1-5 shows the employment growth trends and forecast in 2030 and 2050…………………………..18

Figure 1-1-6 Trip Assignment for the horizon year - 2030……………………………………………………..21

Figure 1-1-7 : Trip Assignment for the horizon year – 2050…………………………………………….……22

Figure 2-1: Relation between Urban rail and City population	24

Figure 2-2:  Expected Metro Network Length in Delhi in relation to population	24

Figure 2-3 MRTS and BRTS corridor in Delhi	25

Figure 2-4Trip Assignment for the horizon year - 2030	27

Figure 2-5 Trip Assignment for the horizon year – 2050	27

Figure 2-6Trip Assignment for the horizon year - 2030	31

Figure 2-7 Trip Assignment for the horizon year – 2050	31

Figure 2-8Trip Assignment for the horizon year - 2030	33

Figure 2-9 Trip Assignment for the horizon year – 2050	34

Figure 2.10: Past trend of petrol price……………………………………………………………………………35

Figure 2-11: Past Trend of Diesel Price	36

Figure 2-12: Past trend of CNG Price	36

Figure 2-13 Trip Assignment for the horizon year - 2030	38

Figure 2-14  Trip Assignment for the horizon year – 2050	38

Figure 2-15 Trip Assignment for the horizon year - 2030	41

Figure 2-16 Trip Assignment for the horizon year – 2050	41

Figure 2-17 Trip Assignment for the horizon year – 2050	43

Figure 2-18 Trip Assignment for the horizon year – 2050	46

Figure 3-1: Comparison of Co2 Emissions of various scenarios	52

Figure 3-2 : Emissions comparison of Base year & BAU Scenario	52

Figure 3-3 : Emissions comparison of Policy Bundle 1: Planning	53

Figure 3-4 : Emissions comparison of Policy Bundle 2: Travel demand Management (TDM)	53

Figure 3-5 : Emissions comparison of Policy Bundle 3: Electric Mobility	54

Figure 3-6 : Emissions comparison of Policy Bundle 1 & 2 with Electric Mobility	54

- BAU Scenario

Background

The base year calibrated travel demand model for Delhi has been applied for estimating the future year network performance evaluation between 2030 and 2050. This process enables an assessment of impact of changes in travel pattern caused by change in level of service in the transportation network in horizon year under Business As Usual Scenario (BAU). The process involves running the travel demand model based on projected trip forecasts and future transport network.

Horizon Year Road Network

The horizon year’s transport network includes various committed new road links and mass transit improvement proposals. Besides the existing network, the proposed new road links of 252 Km are as per Public Works Department (PWD) plans .The proposed road improvement links as shown in the Figure 1.1

Figure 1.1: Horizon year Road Network for 2030 & 2050

New committed road network projects in Delhi

The list of committed road projects in Delhi along with their details are shown in the below Table 1.1

Table 1-1: Existing Link Capacities and Speeds for different category of roads

| Sr. No | Road Name | Estimated Year of completion | Length (in Km) |
| --- | --- | --- | --- |
| 1 | North to South corridor | 2018 | 20.10 |
| 2 | East - west Corridor | 2018 | 39.40 |
| 3 | Elevated Road along GT Road | - | 15.00 |
| 4 | Elevated Road along Najafagarh Drain | 2018 | 26.50 |
| 5 | Elevated Road along Nangloi Drain | 2017 | 16.20 |
| 6 | Barapullah Extension Road | 2017-2018 | 15.40 |
| 7 | Kalindi Bypass | - | 12.50 |
| 8 | Elevated Road along Mangal Pandey Road | - | 8.87 |
| 9 | Northern Peripheral Expressway | - | 3.51 |
| 10 | Sec 56 - Mahipalpur Road (Mandi Road) | - | 22.70 |
| 12 | High speed corridor (11 Murti - Airport) | 2017 | 8.22 |
| 13 | Underpass | - | 11.70 |
| 15 | Meharuli - Badrpur road Extension | 2018 | 7.22 |
| 16 | Palam vihar - Anand parbat (supple drain) | 2015 | 9.83 |
| 17 | Along Yamuna western canal | 2018 | 5.99 |
| 18 | Zakhira - Tikri Border (along RL) | 2018 | 19.10 |
| 19 | Lodi road - Ring Road (tunnel) | 2017 | 1.37 |
| 20 | RL Hosiptal - Ridge Road (tunnel) | 2017 | 2.56 |
| 21 | Elevated road along Shahdara drain | - | 5.57 |
| Total |  |  | 251.74 |

Horizon year Metro system network

Besides the existing metro network, the proposed new metro route length is 260.88 Km as per the Metro network augmentation proposals by DMRC.

Proposed Metro Network in Delhi (Phase - III)

While the deadline for completion of Phase 3 was 2016, however, it is expected that the entire Phase III will be complete by September 2017. Out of 2 new lines and 11 route extensions proposed for Phase III, cabinet approved for 2 new lines and 10 route extensions totalling 167.27 km. Construction has already begun on many of these. In April 2014, the Delhi Lt. Governor gave approval for two further extensions. The additional metro line phase III details are shown in below Table 1.2:

Table 1-2: Additional Metro Lines in Phase - III

| Line | Stations | Length (in KM) | Terminals |  | No. of interchanges planned |
| --- | --- | --- | --- | --- | --- |
| Violet Line extension | 7 | 9.37 | Central Secretariat | Kashmere Gate | 2 |
|  | 9 | 17 | Badarpur | Ballabhgarh | 0 |
| Blue Line extension | 3 | 4.295 | Dwarka | Najafgarh | 1 |
|  | 6 | 6.675 | Noida City Centre | Noida Electronic City | 1 |
| Green Line extension | 6 | 11.182 | Mundka | Bahadurgarh | 0 |
| Pink Line also called Inner Ring Road Line (Line 7) | 38 | 58.59 | Mukundpur | Shiv Vihar | 11 |
| Magenta Line also called Outer Ring Road Line (Line 8) | 25 | 38.24 | Janakpuri West | Botanical Garden | 4 |
| Red Line extension | 6 | 9.6 | Dilshad Garden | New Bus Stand, Ghaziabad | 0 |
| Total | 103 | 154.952 |  |  | 18 |

Source: DMRC

Proposed Metro Network in Delhi (Phase - IV)

Phase IV has a 2021 deadline, and tentatively includes further extensions to Sonia Vihar, Burari, Mukundpur, Reola Khanpur, Palam, Najafgarh, Narela, Ghazipur, Noida sector 62, extensions of Violet line, Green line, Line 8, having a total length of over 100 km. All the lines of phase IV are shown in the below Table 1.3.

Table 1-3: Additional Metro Lines in Phase - IV

| Line | Stations | Length (in KM) | Terminals |  | No. of interchanges planned |
| --- | --- | --- | --- | --- | --- |
| Aqua Line | 10 | 12.58 | Inderlok | Indraprastha | 5 |
| Moss Green | 16 | 22.2 | Tughlakabad | IGI Terminal 1 | 4 |
| Moss Green extension | 6 | 7.96 | Saket-G Block | Lajpat Nagar | 3 |
| Pink Line also called Inner Ring Road Line (Line 7) | 7 | 12.54 | Mukundpur | Maujpur |  |
| Magenta Line also called Outer Ring Road Line (Line 8) | 26 | 28.92 | Janakpuri West | Ramakrishna Ashram Marg | 9 |
| Red Line extension | 16 | 21.73 | Rithala | Narela | 0 |
| Total | 81 | 105.93 |  |  | 21 |

The combined phase–III & Phase-IV metro network is shown in Figure 1.2 below

Figure 1.2 : Horizon year Metro Network for 2030 & 2050

Network Speed Assumptions

Speeds for upgraded/new horizon year road network were based on its lane configuration and capacity as recommended in Indian Roads Congress (IRC – 86) guidelines. Various road categories were identified for different road types and speeds and capacities were estimated for each of these road types. Average speeds for each road type were estimated based on design speeds & factoring LOS-‘C’. Table 1.4 shows capacities and speed assumptions which were considered for this study.

Table 1-4: Existing Link Capacities and Speeds for different category of roads

| Sr. No | St Type | Capacity (PCU) | Direction Capacity (PCU) | Speed (Kph) |
| --- | --- | --- | --- | --- |
| 1 | AR 2LUD | 1500 | 750 | 40 |
| 2 | AR 4LD | 3600 | 1800 | 50 |
| 3 | AR 6LD | 5400 | 2700 | 50 |
| 4 | AR 8LD | 7200 | 3600 | 60 |
| 5 | SAR 2LUD | 1200 | 600 | 35 |
| 6 | SAR 4LD | 2900 | 1450 | 45 |
| 7 | SAR 6LD | 4300 | 2150 | 45 |
| 8 | CS 2LUD | 900 | 450 | 30 |

AR: Arterial Road, SAR: Sub-Arterial Road, CS: Collector Streets; UD: Undivided, LD: Lane Divided

The operating speed for metro system is based on existing pattern of operations in metro network as provided in the operating manual of DMRC

Planning Forecast

In Delhi, all the planning provision has been followed the two level, first is city level - Master Plan Delhi (MPD 1981, 2001 and 2021) and Second Planning Zone level – 16 Zonal Plans for Delhi. By taking the same approach, all the forecast for planning parameters for BAU scenario is undertaken at the three level, first City Level, second Planning Zone level and  third TAZ Level respectively..

Population Projections

1.5.1.1 City Level Forecast

Before attempting to forecast Delhi’s population projection for the year 2030 & 2050 have been made to assess the population growth trends in other mega cities of the country namely Mumbai, Kolkata & Chennai. Trends of population growth between (1941–2011) is observed to decline in Delhi similar to all mega cities  namely Mumbai, Chennai and Kolkata (Table 1.5)

Table 1-5: Comparison of population growth in Delhi with similar metropolitan cities in India

| Year | Delhi | Gr. Mumbai (UA) | Kolkata | Chennai | CAGR Delhi | CAGR Gr. Mumbai(UA) | CAGR Kolkata | CAGR Chennai |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1941 | 9.17 | 18.1 | 36.21 | 7.7 |  |  |  |  |
| 1951 | 17.4 | 33.1 | 46.69 | 14.16 | 6.64% | 6.22% | 2.57% | 6.28% |
| 1961 | 26.6 | 46.6 | 59.83 | 17.29 | 4.30% | 3.48% | 2.51% | 2.02% |
| 1971 | 40.7 | 65.92 | 74.2 | 24.69 | 4.34% | 3.53% | 2.18% | 3.63% |
| 1981 | 62.2 | 94.22 | 91.94 | 32.66 | 4.35% | 3.64% | 2.17% | 2.84% |
| 1991 | 94.2 | 125.72 | 109.1 | 38.41 | 4.24% | 2.93% | 1.73% | 1.63% |
| 2001 | 137.8 | 163.68 | 132.17 | 43.43 | 3.88% | 2.67% | 1.94% | 1.24% |
| 2011 | 167.5 | 184.14 | 140.35 | 46.81 | 1.97% | 1.18% | 0.60% | 0.75% |

Source:  Census of India

It is observed that in case of Delhi like other cities there  would be decline in population growth rate till 2030 & 2050 .For the horizon year 2030 and 2050, the population is forecasted by using the past growth trend of population from (1951 to 2011). based on various methods like linear trend fit, geometric progress, incremental, exponential and CAGR (compound aggregated growth rate).  Amongst the various methods the estimate by geometric & exponential method trend fitting method gives very high estimates of population i.e 38.74 million & 51.62 million in 2030 and 78.54 million & 127.60 million in 2050 respectively and thus have been rejected. The estimate obtained from arithmetic method growth rate method of 21.2 million in 2030 & 26.23 million in 2050 appears on lower side and thus has not been considered.

The following table 1.6 shows the population estimates by linear trend method, incremental growth rate and compound annual growth rate method respectively.

Table 1-6: Estimated Population by various methods

| Years | Linear Trend | Annual GR (%) | Incremental | Annual GR (%) | CAGR | Annual GR (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 2021 | 21.38 | 2.44% | 21.41 | 2.45% | 23.88 | 3.58% |
| 2030 | 24.85 | 1.68% | 25.92 | 2.15% | 28.37 | 1.93% |
| 2040 | 28.70 | 1.45% | 31.78 | 2.06% | 33.36 | 1.63% |
| 2050 | 32.55 | 1.27% | 38.50 | 1.94% | 38.34 | 1.40% |

It is observed that linear trend method estimate for year 2030 & 2050 respectively are lowest and are in continuity with the observed declining growth rate trend of population and there have been adopted. This method gives estimate of 21.38 million in 2021 which is also closest to Delhi’s Master Plan population in 2021 after discounting for excess difference from their projected value of 1.2 million from their projected value of 23.0 million. The difference in population is based on 2011 population forecasts of MPD-2011 of 18.0 million and actual census population value of 16.80 million and has been assumed to be applicable in year 2021 as well.

Keeping in view the fact there were be declining trend of population growth in Delhi in horizon years 2030 & 2050. It is concluded that those population estimates were logical which besides fitting the past trends also provide the lower estimate of the population.

The population estimate in the present study are also in continuity with estimates of Global cities Institute working paper-2014, which estimates Delhi’s population in the year 2025 & 2050 to be 22.68 million & 31.64 million respectively (Table 1.7)

Table 1-7: Population Estimations by Population Reference Bureau report 2007

| Global cities institute working paper no. 4 (study 2014) |  |  |
| --- | --- | --- |
| Year | Population | CAGR |
| 2010 | 17,015,000 |  |
| 2025 | 22,680,000 | 1.93% |
| 2050 | 31,638,000 | 1.34% |
| 2075 | 38,719,000 | 0.81% |
| 2100 | 44,465,000 | 0.56% |

Source: Global Institute Working Paper, 2014

- 1.5.1.2 Planning Zone Level Forecast

There are 16 planning zones in Delhi comprising 8 internal zones while 8 are urban extensions. Figure 1.3 shows the planning zone system in Delhi.

Figure 1.3: Map of Planning Zones in Delhi

The population and area under each planning zone for the year 1981, 1991, 2001 and 2011 are presented below in the Table 1.8.

Table 1.8: Population and Area under various planning zones

| Planning Zone | Population (Census) |  |  |  | Area |  | Density |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1981 | 1991 | 2001 | 2011 | Residential Area (Ha.) | Area (Ha) | Residential Density PPH | Gross Density PPH |
| A | 6,84,001 | 6,37,712 | 5,70,000 | 5,61,224 | 437 | 1,198 | 1284 | 468 |
| B | 5,67,804 | 5,81,000 | 6,19,200 | 6,38,827 | 1,031 | 2,852 | 620 | 224 |
| C | 5,30,547 | 5,53,089 | 6,79,000 | 7,77,753 | 1,627 | 4,146 | 478 | 188 |
| D | 4,96,058 | 5,87,207 | 5,87,000 | 5,78,203 | 2,444 | 6,481 | 237 | 89 |
| E | 10,28,794 | 21,54,905 | 27,98,000 | 38,77,142 | 5,653 | 8,819 | 686 | 440 |
| F | 8,21,800 | 12,09,100 | 17,17,000 | 18,84,477 | 4,236 | 11,973 | 445 | 157 |
| G | 8,68,277 | 12,29,607 | 16,29,122 | 17,45,958 | 3,730 | 11,073 | 468 | 158 |
| H | - | - | 12,26,000 | 16,54,040 | 2,757 | 5,315 | 600 | 311 |
| J | - | 2,37,455 | 4,07,745 | 9,27,401 | 4,547 | 15,143 | 204 | 61 |
| K I | - | - | 2,38,258 | 8,54,295 | 3,464 | 4,100 | 247 | 208 |
| K II | - | - | 3,30,000 | 10,63,800 | 2,957 | 6,805 | 360 | 156 |
| L | 88,342 | 1,57,561 | 2,02,696 | 3,12,664 | 5,344 | 24,788 | 59 | 13 |
| M | - | 80,579 | 2,84,000 | 7,78,428 | 3,235 | 5,516 | 241 | 141 |
| N | - | - | 1,41,217 | 3,04,528 | 4,704 | 14,800 | 65 | 21 |
| P I | - | 1,07,204 | 1,45,618 | 3,24,109 | 3,164 | 10,398 | 102 | 31 |
| P II | - | - | 3,57,601 | 5,05,093 | 3,293 | 6,719 | 153 | 75 |
| Total |  |  | 1,19,32,457 | 1,67,87,941 | 52,621 | 1,40,126 | 319 | 120 |

Source: Census of India and Master Plan Delhi 2021

MPD 2021

According to MPD 2021, existing residential areas may provide a potential to accommodate about 15.3 million population ultimately i.e. 11.4 million in Zones A to H and 3.9 million in Dwarka, Rohini Phase III, IV & V and Narela. Out of the remaining 7.7 million population, 2.9 million already exists in villages, census towns, unauthorised colonies and JJ clusters in the present rural areas. Here about 4.8 million additional population is to be accommodated in the future urban extensions. The proposed and revised holding capacity for the year 2021 in the planning zone are presented below in the Table 1-9.

Table 1-9: Proposed and Revised Holding Capacity in Planning Zone for the year 2021 (‘000)

| Zone | Holding capacity | Existing population | Holding capacity |
| --- | --- | --- | --- |
|  | MPD 2001 | 2001 | 2021 |
| A | 420 | 570 | 570 |
| B | 630 | 624 | 630 |
| C | 751 | 679 | 788 |
| D | 755 | 587 | 813 |
| E | 1,789 | 2,798 | 2,800 |
| F | 1,278 | 1,717 | 1,975 |
| G | 1,490 | 1,629 | 1,955 |
| H | 1,865 | 1,226 | 1,865 |
| J |  | 408 | 2,067 |
| K I |  | 238 | 1,200 |
| K II |  | 330 | 1,300 |
| L |  | 203 | 2,000 |
| M |  | 284 | 980 |
| N |  | 141 | 2,418 |
| P I |  | 146 | 1,620 |
| P II |  | 358 | 1,900 |

Source: Master Plan Delhi 2021

The population estimation by MPD 2021 for the year 2011 was 18 million while for the 2021 it was 23 million. Same population distribution has been adopted at planning zone level also. As per projection by SPA Delhi by the year 2021 population estimation for Delhi is 21.38 million.

- Zone wise population estimation

For forecasting population it has been assumed that the population in divisions A, B, C and D show a declining trend as observed from the past trend. The special character of division C (civil lines) and D (New Delhi) which housings most government (central & Delhi govt.) offices which has restricted potential for growth. Further division A (walled city) & division B (city extension – Karol Bagh). In particular division A (walled city) and division D (New Delhi) have restricted potential for residential development owing to saturation density level planned regulated development respectively. Division B (city extension- Karol Bagh) and to some extent Division C (civil lines) will show moderate increase owing to intense built up development and low density regulated controlled development respectively.

The distribution of remaining population of 18.7 million will primarily has been assumed to be distributed areas reaming zones falling into cognizance the existing pattern of population distribution. The temporal trends of planning division wise population for 1991-2011 are shown in Table 1.10 below.

Table 1.10: Population of Planning Zones from 1991 to 2011

| Planning Zone | 1991 | 2001 | 2011 |
| --- | --- | --- | --- |
| A | 637,712 | 570,000 | 561,224 |
| B | 581,000 | 619,200 | 638,827 |
| C | 553,089 | 679,000 | 777,753 |
| D | 587,207 | 587,000 | 578,203 |
| E | 2,154,905 | 2,798,000 | 3,877,142 |
| F | 1,209,100 | 1,717,000 | 1,884,477 |
| G | 1,229,607 | 1,629,122 | 1,745,958 |
| H |  | 1,226,000 | 1,654,040 |
| J | 237,455 | 407,745 | 927,401 |
| K I |  | 238,258 | 854,295 |
| K II |  | 330,000 | 1,063,800 |
| L | 157,561 | 202,696 | 312,664 |
| M | 80,579 | 284,000 | 778,428 |
| N |  | 141,217 | 304,528 |
| P I | 107,204 | 145,618 | 324,109 |
| P II |  | 357,601 | 505,093 |
| Total | 7,535,419 | 11,932,457 | 16,787,941 |

Table 1.11 shows the forecasted population estimates.

Table 1.11: Population Estimation at Planning Zonal level for Horizon Year 2030 and 2050

| Planning Zone | 2011 | 2021 | 2030 | 2050 |
| --- | --- | --- | --- | --- |
| A | 5,61,224 | 5,13,157 | 4,78,738 | 4,02,250 |
| B | 6,38,827 | 6,70,836 | 6,96,858 | 7,54,685 |
| C | 7,77,753 | 8,94,611 | 9,30,057 | 11,03,563 |
| D | 5,78,203 | 5,69,406 | 5,61,489 | 5,43,895 |
| E | 38,77,142 | 47,53,953 | 53,85,871 | 68,45,951 |
| F | 18,84,477 | 23,28,261 | 25,68,011 | 31,37,888 |
| G | 17,45,958 | 21,22,901 | 23,12,242 | 27,70,922 |
| H | 16,54,040 | 20,78,673 | 23,77,780 | 30,63,678 |
| J | 9,27,401 | 12,12,160 | 14,69,295 | 20,41,503 |
| K I | 8,54,295 | 14,67,925 | 19,51,288 | 30,02,320 |
| K II | 10,63,800 | 17,94,658 | 23,68,821 | 36,18,838 |
| L | 3,12,664 | 3,69,236 | 4,18,704 | 5,32,914 |
| M | 7,78,428 | 10,77,086 | 13,42,337 | 19,27,345 |
| N | 3,04,528 | 4,67,073 | 5,92,508 | 8,67,868 |
| P I | 3,24,109 | 4,08,545 | 6,39,173 | 9,40,493 |
| P II | 5,05,093 | 6,51,517 | 7,56,829 | 9,95,887 |
| Total | 1,67,87,941 | 2,13,80,000 | 2,48,50,000 | 3,25,50,000 |

Fig.1.4 shows graphically the population growth trends in past and likely estimates in 2030 and 2050

Fig. 1.4: Population Growth Trends and Projections in Delhi

The proposed residential density at planning zonal level for the Horizon year 2030 and 2050 is presented below in the Table 1.12

.

Table1.12: Proposed residential density in each planning zone a for horizon year 2030 and 2050

| Planning Zone | 2011 | 2021 | 2030 | 2050 |
| --- | --- | --- | --- | --- |
| A | 1,284 | 1,174 | 1,096 | 920 |
| B | 620 | 651 | 676 | 732 |
| C | 478 | 550 | 572 | 678 |
| D | 237 | 233 | 230 | 223 |
| E | 686 | 841 | 953 | 1,211 |
| F | 445 | 550 | 606 | 741 |
| G | 468 | 569 | 620 | 743 |
| H | 600 | 754 | 863 | 1,111 |
| J | 204 | 267 | 323 | 449 |
| K I | 247 | 424 | 563 | 867 |
| K II | 360 | 607 | 801 | 1,224 |
| L | 59 | 69 | 78 | 100 |
| M | 241 | 333 | 415 | 596 |
| N | 65 | 99 | 126 | 184 |
| P I | 102 | 129 | 202 | 297 |
| P II | 153 | 198 | 230 | 302 |
| Total | 319 | 406 | 472 | 619 |

      - TAZ Level Forecast

For the population estimation at TAZ level, the distribution of population within TAZ in RITES (2007) study has been taken into consideration ensuring the sum of population of TAZs under each planning zone does not exceed the total of that respective planning zone.

1.5.2 Employment forecast

As per 2011 Census, Delhi had an employment size of 5.58 million. The workforce participation for Delhi has increased from 31.64% in 1991 to 33.3% 2001. (Table 1.13)

Table 1.13: Work force participation in Delhi (in Lakhs)

| Years | Total Workers | Workforce Participation |
| --- | --- | --- |
| 1961 | 0.85 | 32.14% |
| 1971 | 12.28 | 30.21% |
| 1981 | 20.02 | 32.19% |
| 1991 | 29.8 | 31.63% |
| 2001 | 45.45 | 32.82% |
| 2011 | 55.87 | 33.28% |

Source: Census of India.

1.5.2.1 City Level Forecast

As Delhi is almost a fully urbanized state in the Indian, it is assumed workforce participation will grow at the same rate for future as observed from the part trend (1981- 2011) .

By taking the estimation of an increase of 0.8% in decadal growth of workforce participation from the previous years’ workforce participation, the WFPR for horizon year 2030 and 2050 has been estimated to be 35.07% and 36.70% respectively. Workforce participation rates are presented in Table 1.14

Table 1.14: Estimated workforce participation rate for the year 2030 and 2050

| Year | Workforce Participation |
| --- | --- |
| 1981 | 32.19 |
| 1991 | 31.63 |
| 2001 | 32.82 |
| 2011 | 33.31 |
| 2021 (P) | 34.27 |
| 2030 (P) | 35.02 |
| 2040 (P) | 35.86 |
| 2050 (P) | 36.70 |

Fig. 1.5 shows the employment forecast for 2030 and 2050

Fig. 1.5 shows the employment growth trends and forecast in 2030 and 2050

Given the estimated workforce participation and population estimates for the horizon year 2030 and 2050, the estimated workers in Delhi in future is shown in the below Table 1.15.

Table 1-15: Estimated Workers for the Horizon Year 2030 and 2050

| HY | Population (in Million) | Employment (in Million) |
| --- | --- | --- |
| 2011 | 16.78 | 5.59 |
| 2021 (P) | 20.73 | 7.10 |
| 2030 (P) | 21.38 | 7.49 |
| 2040 (P) | 24.85 | 8.91 |
| 2050 (P) | 32.55 | 11.95 |

      - Planning Zone Level Forecast

For the year 2021 and 2030, the distribution of projected employment in each planning division is assumed to follow the same distribution as in year 2007 observed in RITES 2008 study. It is assumed that natural growth in employment will take place in internal zones while the employment increase will be more towards outer zone (Urban Extensions). The employment estimations at planning zone level for the horizon year 2021 presented in the Table 1.16.

Table 1.16: Employment estimations at planning zone level for the horizon year 2030 and 2050

| Zones | 2011 | 2021 | 2030 | 2050 |
| --- | --- | --- | --- | --- |
| A | 3,35,047 | 3,08,251 | 2,70,742 | 3,25,130 |
| B | 3,38,748 | 3,11,529 | 2,73,709 | 3,28,809 |
| C | 3,68,352 | 3,40,248 | 2,99,384 | 3,60,976 |
| D | 5,38,855 | 5,02,253 | 4,43,842 | 5,40,177 |
| E | 7,36,633 | 6,93,218 | 6,16,098 | 7,58,405 |
| F | 6,31,935 | 5,87,650 | 5,17,712 | 6,26,726 |
| G | 6,22,972 | 5,82,234 | 5,15,434 | 6,29,495 |
| H | 4,01,162 | 3,79,590 | 3,39,211 | 4,21,583 |
| J | 2,43,368 | 4,03,713 | 4,83,996 | 8,79,543 |
| K I | 2,08,877 | 2,33,965 | 2,47,919 | 3,89,635 |
| K II | 2,67,492 | 3,24,854 | 3,51,808 | 5,71,068 |
| L | 1,96,562 | 8,36,793 | 10,96,097 | 22,01,683 |
| M | 2,55,189 | 3,95,160 | 4,75,780 | 8,61,722 |
| N | 1,63,541 | 4,65,672 | 6,10,300 | 12,09,119 |
| P I | 1,65,671 | 5,01,927 | 6,55,356 | 12,99,153 |
| P II | 1,14,943 | 2,37,654 | 2,90,457 | 5,43,493 |
| Total | 55,89,347 | 71,04,710 | 74,87,846 | 1,19,46,718 |

      - TAZ level Forecast

Employment estimation at the TAZ level is assumed to follow the same pattern as the estimated for 2007, 2011 and 2021 in RITES 2008 study. For the horizon year 2030 and 2050, it is assumed that the past year growth trend shall continue in each zone ensuring that the sum of employment in TAZs under each Planning zone does not exceed the total of that respective zone proposed at planning zone level.

Transport demand forecast

1.6.1 Trip Generation

For the horizon year 2030 and 2050, the per capita trip rate (PCTR) for Delhi has been forecasted based on population growth trend from 1969 to 2001. It is estimated that the vehicular trip rate is likely to reach 1.25 & 1.45 & in the year 2030 & 2050 respectively  (Table 1.17)

Table 1-17: Estimated Motorized PCTR for the cardinal year details

| Year | Population | Motorized PCTR (Trend line) |
| --- | --- | --- |
| 1969 | 3.77 | 0.49 |
| 1981 | 6.20 | 0.72 |
| 2001 | 13.9 | 0.87 |
| 2030 | 24.85 | 1.25 |
| 2050 | 32.55 | 1.45 |

Accordingly the trip ends have been forecasted for 2030 & 2050 respectively taking into account base year trip end model suitably moderated to account for forecasted PCTR values.

    - Trip Distribution

The forecasted trip ends were distributed using the base year calibrated distribution parameters explained in base year travel demand model and horizon year network system attributes. The forecasted average trip length for the year 2030 and 2050 are shown in Table 1.18

Table 1-18: Comparison of Average trip length (ATL) in cardinal years

|  | Base | 2030 | 2050 |
| --- | --- | --- | --- |
| ATL (in KM) | 9.1 | 12.81 | 13.41 |

    - Modal Split

Base year modal split calibrated parameters are used to develop and estimate the future year 2030 and 2050 modal share. It is assumed that modal speeds on network are going to remain same as in base year due to incremental improvements in the network. It is observed that there is a change in public transport from base year 2011 (50%) to 2030 (45%) with a negative shift of -5%.and 2050 (39%) with -11%. For the horizon year 2050, public transport share decreases by 6% compared to 2030 as there is no planned  transit network proposed beyond 2030. Table 1.19 shows the mode wise percentage share of trips in the cardinal years 2030 and 2050.

Table 1-19: Comparison of mode split in Base year and BAU-2030 & 2050 (Per Day)

| Mode | Base Year Trips | % | Trips - 2030 | % | Trips - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- |
| Car | 2487177 | 21% | 7846755 | 22% | 11420266 | 24% |
| Put | 6014525 | 50% | 16053871 | 45% | 18512527 | 39% |
| Auto | 800572 | 7% | 2436316 | 7% | 3220787 | 7% |
| Tw | 2732344 | 23% | 9695560 | 27% | 14043915 | 30% |
| Total | 12034618 | 100% | 36032503 | 100% | 47197496 | 100% |

Table 1.20 shows comparison of modal split in base year and BAU

Table 1-20: Comparison of mode split in Base year and BAU-2030 & 2050 (Per Day)

| Mode |  | Base Year | % | Trips - 2030 | % | Trips - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Car | HB | 1178997 | 21% | 3754502 | 23% | 5523972 | 26% |
| Put |  | 2298291 | 41% | 5223359 | 32% | 5059888 | 24% |
| Auto |  | 141347 | 3% | 367309 | 2% | 460525 | 2% |
| Tw |  | 1963354 | 35% | 7055831 | 43% | 10438613 | 49% |
| Total |  | 5581989 | 100% | 16401001 | 100% | 21482998 | 100% |
| Car | NHB | 1308180 | 20% | 4092253 | 21% | 5896294 | 23% |
| Put |  | 3716234 | 58% | 10830512 | 55% | 13452639 | 52% |
| Auto |  | 659225 | 10% | 2069007 | 11% | 2760262 | 11% |
| Tw |  | 768990 | 12% | 2639729 | 13% | 3605302 | 14% |
| Total |  | 6452629 | 100% | 19631502 | 100% | 25714498 | 100% |

    - Trip Assignment

The horizon year trips for 2030 and 2050 were assigned on the proposed transport network and congested links were identified as shown in the Fig. 1.6  and Fig 1.7 below:

Figure 1.6 Trip Assignment for the horizon year - 2030

Figure 1.7 : Trip Assignment for the horizon year – 2050

The comparative evaluation of alternative networks has been carried out from the point of view of passenger transport demand for the horizon year and ability of the various networks to cater to this demand. Following criterion for comparative evaluation have been considered:

- Average speed of the network

- Passengers trips

- Passenger kilometres

- Passenger hours

- Passengers carried per km; and

Table 1.21 shows the comparisons of base year and BAU scenarios. It can be seen that the average speed shall reduce from 14.9Kph in base year to 5.5Kph & 1.4Kph respectively in 2030 & 2050. There is sustainable increasing in vehicle hours & vehicle kilometers travelled in future with implication in economic loss. The share of public transport (bus, metro, IPT) reduces from 51.3% of base year to 45% & 39% respectively in 2030 & 2050.

Table 1-21: Comparison of Base year and BAU-2030 & 2050 scenario (Peak Hour )

| Transport Network Scenario | Veh Dist Travelled (in Km) | Veh Travelled Hours | Average Speed (Kmph) | % PT+IPT Share | Pax (in No's) | Pax Travelled Km | Pax Travelled Hours | Co2 (in Tonnes) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base Year | 2,465,650 | 165,583 | 14.9 | 50% | 434,197 | 3,426,571 | 101,350 | 230 |
| BAU - 2030 | 11,316,713 | 2,064,762 | 5.5 | 45% | 1,213,903 | 10,911,894 | 334,778 | 946 |
| BAU - 2050 | 19,821,085 | 13,671,726 | 1.4 | 39% | 1,385,215 | 12,974,539 | 402,913 | 1642 |

- Policy Bundles

For this study, three broad level police bundles were considered for evaluation in terms of GHG emissions. Under these broad levels policy bundles there are sub level polices. The list of adopted broad policy bundles and sub polices are presented below:

- Police Bundle 1 : Planning

- Increase in Transit Network

- Promoting Transit Oriented Development (TOD)

- Combination of Increase in Transit Network + Transit Oriented Development

- Police Bundle 2:  Travel Demand Management

- Increase in Fuel Cost

- Congestion Pricing

- Shared Mobility ( Car Pooling + Increase % share of App based Cabs)

- Combination of Fuel cost + Congestion Pricing +  Shared mobility

- Police Bundle 3: Technology

- Electric Mobility

Police Bundle 1: Planning

Policy measure : Increase in Transit Network

Background

In this scenario, additional transit network has been considered to encourage the public transport which will also directly reduce the emissions under the transportation sector within the city. The details of this scenario is explained in detail in the below sections

Relationship between Urban Rail and City Population

From the study of Margaret E.L.Y on “Urban rail networks” based on  Mass rapid transit system (MRTS) and Bus rapid transit system (BRTS)  in world cities it is observed that as city population increases the length of urban rail length also increases (Figure 2.1 ).

Figure 2-1: Relation between Urban rail and City population

Source: Urban rail networks in world cities – Margaret ELY

Using the above relationship between population and urban rail length as observed globally in context for Delhi it is observed that the metro network length in Delhi is likely to reach 459.4 km. and 620 km. in 2030 and 2050 respectively at estimated population levels of 248 lakhs and 326 lakhs respectively ( Fig .2.2)

Figure 2.2:  Expected Metro Network Length in Delhi in relation to population

Transit Network

In addition to BAU scenario network the additional network length of metro and BRTS is shown in Figure 2.3 below which takes into account the future population areas as per the forecasts at TAZ level

Figure 2.3 MRTS and BRTS corridor in Delhi

Population and Employment Forecast

From the planning forecast described in earlier section under BAU scenario the population will increase to 24.8 million and 32.55 million while employment will increase to 8.91 million and 11.95 million in 2030 and 2050 respectively.

Modal Split

Base year calibrated parameters coupled with horizon year public transit network attributes are used to develop and estimate the future year 2030 and 2050 modal share. The estimate shows that there is a change in Home based work public transport share from 32% in 2030 to 43% under increase in transit scenario. Similarly the share increase from 24% in 2050 to 50% respectively .Table 2.1 shows the mode wise percentage share of trips in the cardinal years 2030 and 2050.

Table 2.1 Comparison of mode split in BAU - 2030 & 2050 ( Per Day)

| Mode |  | BAU - 2030 | % | Transit - 2030 | % | BAU - 2050 | % | Transit - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | HB | 3754502 | 23% | 3137591 | 19% | 5523972 | 26% | 3693816 | 17% |
| Public |  | 5223359 | 32% | 7109168 | 43% | 5059888 | 24% | 10888030 | 50% |
| Auto |  | 367309 | 2.2% | 348529 | 2.1% | 460525 | 2.1% | 413960 | 2% |
| Tw |  | 7055831 | 43% | 5805713 | 35% | 10438613 | 49% | 6663318 | 31% |
| Total |  | 16401001 | 100% | 16401001 | 100% | 21482998 | 100% | 21659125 | 100% |
| Car | NHB | 4092253 | 21% | 3278086 | 17% | 5896294 | 23% | 3628972 | 14% |
| Public |  | 10830512 | 55% | 12446694 | 63% | 13452639 | 52% | 17772677 | 70% |
| Auto |  | 2069007 | 11% | 1751495 | 9% | 2760262 | 11% | 1910658 | 7% |
| Tw |  | 2639729 | 13% | 2155227 | 11% | 3605302 | 14% | 2226064 | 9% |
| Total |  | 19631502 | 100% | 19631502 | 100% | 25714498 | 100% | 25538371 | 100% |

Table 2.2 shows the comparison of modal share in BAU and Increase in Transit scenarios. It is observed that Increased transit policy scenario shows a higher modal share in comparison to BAU scenarios ie. Increase from 45% to 54% in 2030 and from 39% to 61% in 2050 respectively

Table 2 .2 : Comparison of mode split in BAU and Increase in Transit-2030 & 2050 (Per Day)

| Mode | BAU - 2030 | % | Trips - 2030 | % | BAU - 2050 | % | Trips - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | 7846755 | 22% | 6415677 | 18% | 11420266 | 24% | 7322788 | 16% |
| Public | 16053871 | 45% | 19555862 | 54% | 18512527 | 39% | 28660707 | 61% |
| Auto | 2436316 | 7% | 2100025 | 6% | 3220787 | 7% | 2324618 | 5% |
| Tw | 9695560 | 27% | 7960940 | 22% | 14043915 | 30% | 8889382 | 19% |
| Total | 36032503 | 100% | 36032503 | 100% | 47197496 | 100% | 47197496 | 100% |

Trip Assignment

The horizon year trips for 2030 and 2050 were assigned on the proposed transport network and congested links were identified as shown in the Fig. 2.4 and  Fig.2.5 below:

Figure 2.4 Trip Assignment for the horizon year - 2030

Figure 2.5 Trip Assignment for the horizon year – 2050

The comparative evaluation of alternative networks has been carried out from the point of view of passenger transport demand for the horizon year and ability of the various networks to cater to this demand. Table 2.3 shows the evaluation of BAU and increase in transit scenario

Table 2.3 : Comparative evaluation  of BAU & Increase in Transit scenario- 2030 & 2050  (Peak Hour)

| Transport Network Scenario | Veh. Dist Travelled (in Km) | Veh Travelled Hours | Average Speed (Kmph) | % PT+IPT Share | Pax. (in No's) | Pax. Travelled Km | Pax. Travelled Hours | Co2 (in Tonnes) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base Year | 2,465,650 | 165,583 | 14.9 | 50% | 434,197 | 3,426,571 | 101,350 | 230 |
| BAU - 2030 | 11,316,713 | 2,064,762 | 5.5 | 45% | 1,213,903 | 10,911,894 | 334,778 | 946 |
| BAU - 2050 | 19,821,085 | 13,671,726 | 1.4 | 39% | 1,385,215 | 12,974,539 | 402,913 | 1642 |
| Increase in Transit - 2030 | 8,508,953 | 1,036,666 | 8.2 | 54% | 1,497,529 | 13,321,070 | 402,068 | 742 |
| Increase in Transit - 2050 | 9,973,066 | 2,067,928 | 4.8 | 61% | 2,311,025 | 20,808,769 | 616,009 | 902 |

It is observed that the average speed shall increase from 5.5 kph and 1.4 kph in BAU to 8.2 Kph & 4.8Kph respectively in 2030 & 2050. The share of public transport (bus, metro, IPT) increases from 45% in 2030 and 39% in 2050 to 54% and 61% respectively. There is a decrease in Co2 emissions observed from 946 tonnes to 742 tonnes in 2030 and from 1642 tonnes to 902 tonnes in 2050 respectively

Policy measure : Promote Transit Oriented Development

Background

In this scenario, transit oriented development has been considered to encourage the public transport which will also directly reduce the emissions under the transportation sector and also increase the sustainable modes share  within the city. The details of this scenario is explained in detail in the below sections.. Transit Oriented Development (TOD) concept was adopted around the Mass Transit (MRTS) stations with a radius of 500M.

2.1.2.2 Population and employment distribution around TOD zones

The proposed land use distribution in the TOD Influence area of 500 m around metro stations is derived from the MPD 2021 modified development control norms for TOD Development. Phase wise implementation of TOD is adopted for this study. Planning zone A to D will have the natural growth keeping in view their peculiar character of controlled development due to prevailing regulations while other planning zones will have TOD induced population and employment. To estimate the impact of TOD around the identified metro stations, existing and proposed land uses around the station is estimated based on broad guidelines proposed in Delhi Master Plan. Transit Oriented Development (TOD) will be implemented by encouraging the FAR (Floor Area Ratio) of 2.0 for residential areas and FAR (Floor Area Ratio) 3.0 for commercial and other land uses.

Planning Zone wise Population

| Planning Zone | 2001 | 2011 | 2030 | 2050 |
| --- | --- | --- | --- | --- |
| A | 570,000 | 561,224 | 567,748 | 600,201 |
| B | 619,200 | 638,827 | 779,392 | 933,769 |
| C | 679,000 | 777,753 | 951,571 | 1,156,757 |
| D | 587,000 | 578,203 | 597,258 | 632,416 |
| E | 2,798,000 | 3,877,142 | 5,335,592 | 6,763,903 |
| F | 1,717,000 | 1,884,477 | 2,503,538 | 3,028,961 |
| G | 1,629,122 | 1,745,958 | 2,330,711 | 2,830,713 |
| H | 1,226,000 | 1,654,040 | 2,363,044 | 3,038,717 |
| J | 407,745 | 927,401 | 1,405,732 | 1,904,010 |
| K I | 238,258 | 854,295 | 1,932,805 | 2,925,781 |
| K II | 330,000 | 1,063,800 | 2,258,008 | 3,351,467 |
| L | 202,696 | 312,664 | 453,643 | 604,170 |
| M | 284,000 | 778,428 | 1,316,407 | 1,860,497 |
| N | 141,217 | 304,528 | 597,376 | 869,470 |
| P I | 145,618 | 324,109 | 706,944 | 1,066,881 |
| P II | 357,601 | 505,093 | 750,231 | 982,287 |
| Total | 11,932,457 | 16,787,941 | 24,850,000 | 32,550,000 |

The employment was estimated based on pattern of employment densities (employees per unit of unit of ha of land use)  prevalent in Indian cities in general and Delhi in particular sourced and collated from various in house research studies of SPA Delhi . The estimated planning zone wise employment have been calculated adopting the employment densities and various land uses in each planning zones and shown below:

Planning Zone wise Employment

| Planning Zone | 2007 | 2011 | 2030 | 2050 |
| --- | --- | --- | --- | --- |
| A | 373,380 | 375,468 | 469,170 | 474,812 |
| B | 377,140 | 379,249 | 514,260 | 559,726 |
| C | 414,803 | 413,113 | 491,637 | 557,540 |
| D | 601,740 | 605,104 | 768,467 | 963,563 |
| E | 821,969 | 826,565 | 1,257,938 | 1,432,425 |
| F | 707,937 | 711,895 | 1,074,123 | 1,281,230 |
| G | 695,332 | 699,220 | 1,010,159 | 1,168,981 |
| H | 445,585 | 448,076 | 656,108 | 769,398 |
| J | 173,207 | 174,175 | 407,271 | 686,132 |
| K I | 180,081 | 181,088 | 315,971 | 391,380 |
| K II | 231,415 | 232,709 | 459,709 | 640,578 |
| L | 130,402 | 131,131 | 260,332 | 843,954 |
| M | 164,794 | 165,715 | 351,752 | 536,234 |
| N | 72,394 | 72,799 | 242,137 | 623,145 |
| P I | 83,868 | 84,337 | 232,971 | 641,590 |
| P II | 84,223 | 88,703 | 191,128 | 376,029 |
| Total | 5,558,271 | 5,589,347 | 8,703,133 | 11,946,718 |

Modal Split

Base year calibrated parameters are used to develop and estimate the future year 2030 and 2050 network. The estimate shows that there is a change in public transport from base year 2011 (50%) to 2030 (56%) with a positive shift of +6%. For the horizon year 2050, public transport share is increase +6% with comparing to 2030 as the additional development is considered along the mass transit system & congestion on road increases. Table 2.4  shows the mode wise percentage share of trips in the cardinal years 2030 and 2050.

Table 2-4 Comparison of mode split in BAU and TOD (Home based and Non home based) - 2030 & 2050 (Per Day)

| Mode |  | BAU - 2030 | % | TOD - 2030 | % | BAU - 2050 | % | TOD - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | HB | 3754502 | 23% | 2913315 | 18% | 5523972 | 26% | 3287999 | 15% |
| Put |  | 5223359 | 32% | 7803507 | 48% | 5059888 | 24% | 11970346 | 56% |
| Auto |  | 367309 | 2.2% | 398980 | 2.4% | 460525 | 2.1% | 462091 | 2% |
| Tw |  | 7055831 | 43% | 5285199 | 32% | 10438613 | 49% | 5762562 | 27% |
| Total |  | 16401001 | 100% | 16401001 | 100% | 21482998 | 100% | 21482999 | 100% |
| Car | NHB | 4092253 | 21% | 3316492 | 17% | 5896294 | 23% | 4119503 | 16% |
| Put |  | 10830512 | 55% | 12219155 | 62% | 13452639 | 52% | 17438505 | 68% |
| Auto |  | 2069007 | 11% | 1913169 | 10% | 2760262 | 11% | 2060025 | 8% |
| Tw |  | 2639729 | 13% | 2182684 | 11% | 3605302 | 14% | 2096465 | 8% |
| Total |  | 19631502 | 100% | 19631501 | 100% | 25714498 | 100% | 25714498 | 100% |

Table 2-5: Comparison of mode split in BAU and TOD - 2030 & 2050  (Per Day )

| Mode | BAU - 2030 | % | TOD - 2030 | % | BAU - 2050 | % | TOD - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | 7846755 | 22% | 6229807 | 17% | 11420266 | 24% | 7407503 | 16% |
| Put | 16053871 | 45% | 20022662 | 56% | 18512527 | 39% | 29408851 | 62% |
| Auto | 2436316 | 7% | 2312149 | 6% | 3220787 | 7% | 2522116 | 5% |
| Tw | 9695560 | 27% | 7467883 | 21% | 14043915 | 30% | 7859027 | 17% |
| Total | 36032503 | 100% | 36032501 | 100% | 47197496 | 100% | 47197497 | 100% |

Trip Assignment

The horizon year trips for 2030 and 2050 were assigned on the proposed transport network and congested links were identified as shown in the Fig. 2.6 and Fig 2.7 respectively below:

Figure 2-6 Trip Assignment for the horizon year - 2030

Figure 2-7 Trip Assignment for the horizon year – 2050

The comparative evaluation of alternative networks has been carried out from the point of view of passenger transport demand for the horizon year and ability of the various networks to cater to this demand. Table 2.6 shows the evaluation of BAU and increase in transit scenario

Table 2.6: Comparison of BAU and TOD - 2030 & 2050 scenario (Peak Hour)

| Transport Network Scenario | Veh Dist Travelled (in Km) | Veh Travelled Hours | Average Speed (Kmph) | % PT+IPT Share | Pax (in No's) | Pax Travelled Km | Pax Travelled Hours | Co2 (in Kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base Year | 2,465,650 | 165,583 | 14.9 | 50% | 434,197 | 3,426,571 | 101,350 | 230 |
| BAU - 2030 | 11,316,713 | 2,064,762 | 5.5 | 45% | 1,213,903 | 10,911,894 | 334,778 | 946 |
| BAU - 2050 | 19,821,085 | 13,671,726 | 1.45 | 39% | 1,385,215 | 12,974,539 | 402,913 | 1642 |
| TOD - 2030 | 6,905,254 | 653,359 | 10.6 | 56% | 1,500,347 | 11,831,675 | 352,106 | 606 |
| TOD - 2050 | 7,664,055 | 1,107,692 | 6.9 | 62% | 2,243,830 | 17,980,208 | 542,521 | 726 |

It is observed that the average speed shall increase from 5.5 kph and 1.45 kph in BAU to   10.6 Kph & 6.9 Kph respectively in 2030 & 2050. The share of public transport (bus, metro, IPT) increases from 45% in 2030 and 39% in 2050 to 56% and 62% respectively. There is a decrease in Co2 emissions observed from 946 tonnes to 606 tonnes in 2030 and from 1642 tonnes to 726 tonnes in 2050 respectively.

Policy measure: Combination of Increase in Transit + Transit Oriented Development scenarios

Background

In this scenario, transit oriented development and increase in transit network has been considered to encourage the public transport which will also directly reduce the emissions under the transportation sector and also increase the sustainable modes share within the city. The details of this scenario is explained in detail in the below sections.

Modal Split

Base year calibrated parameters are used to develop and estimate the future year 2030 and 2050 network. The estimate shows that there is a change in public transport from base year 2011 (50%) to 2030 (58%) with a positive shift of +8%. For the horizon year 2050, public transport share is increase +8% with comparing to 2030 as the additional development is considered along the mass transit system & congestion on road increases. Table 2.7 and Table 2.8 shows the mode wise percentage share of trips in the cardinal years 2030 and 2050 and comparison between BAU and alternate policy scenario

Table 2.7 Comparison of mode split ( Home based and non -home based)  in BAU & Transit + TOD - 2030 & 2050 (Per Day)

| Mode |  | BAU - 2030 | % | Trips - 2030 | % | BAU - 2050 | % | Trips - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | HB | 3754502 | 23% | 2926415 | 18% | 5523972 | 26% | 3199677 | 15% |
| Put |  | 5223359 | 32% | 8266259 | 50% | 5059888 | 24% | 13021595 | 61% |
| Auto |  | 367309 | 2.2% | 380716 | 2.3% | 460525 | 2.1% | 438811 | 2% |
| Tw |  | 7055831 | 43% | 4827611 | 29% | 10438613 | 49% | 4822916 | 22% |
| Total |  | 16401001 | 100% | 16401001 | 100% | 21482998 | 100% | 21482999 | 100% |
| Car | NHB | 4092253 | 21% | 3155049 | 16% | 5896294 | 23% | 3018917 | 12% |
| Put |  | 10830512 | 55% | 12513474 | 64% | 13452639 | 52% | 18167725 | 71% |
| Auto |  | 2069007 | 11% | 1835862 | 9% | 2760262 | 11% | 2356186 | 9% |
| Tw |  | 2639729 | 13% | 2127117 | 11% | 3605302 | 14% | 2171671 | 8% |
| Total |  | 19631502 | 100% | 19631501 | 100% | 25714498 | 100% | 25714498 | 100% |

Table 2.8 : Comparison of mode split in BAU & Transit + TOD - 2030 & 2050 (per day)

| Mode | BAU - 2030 | % | Trips - 2030 | % | BAU - 2050 | % | Trips - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | 7846755 | 22% | 6081463 | 17% | 11420266 | 24% | 6218594 | 13% |
| Put | 16053871 | 45% | 20779733 | 58% | 18512527 | 39% | 31189319 | 66% |
| Auto | 2436316 | 7% | 2216578 | 6% | 3220787 | 7% | 2794997 | 6% |
| Tw | 9695560 | 27% | 6954727 | 19% | 14043915 | 30% | 6994586 | 15% |
| Total | 36032503 | 100% | 36032501 | 100% | 47197496 | 100% | 47197497 | 100% |

Trip Assignment

The horizon year trips for 2030 and 2050 were assigned on the proposed transport network and congested links were identified as shown in the fig. 2.8 and fig 2.9 below:

Figure 2-8 Trip Assignment for the horizon year - 2030

Figure 2-9 Trip Assignment for the horizon year – 2050

The comparative evaluation of alternative networks has been carried out from the point of view of passenger transport demand for the horizon year and ability of the various networks to cater to this demand. Table 2.9 shows the evaluation of BAU and increase in transit scenario

Table 2-9: Comparison of Base year, BAU & Transit + TOD - 2030 & 2050 (Peak hour)

| Transport Network Scenario | Veh Dist Travelled (in Km) | Veh Travelled Hours | Average Speed (Kmph) | % PT+IPT Share | Pax (in No's) | Pax Travelled Km | Pax Travelled Hours | Co2 (in Kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base Year | 2,465,650 | 165,583 | 14.9 | 50% | 434,197 | 3,426,571 | 101,350 | 230 |
| BAU - 2030 | 11,316,713 | 2,064,762 | 5.5 | 45% | 1,213,903 | 10,911,894 | 334,778 | 946 |
| BAU - 2050 | 19,821,085 | 13,671,726 | 1.4 | 39% | 1,385,215 | 12,974,539 | 402,913 | 1642 |
| Transit + TOD - 2030 | 6,365,383 | 589,818 | 10.8 | 58% | 1,546,676 | 12,313,139 | 364,058 | 508 |
| Transit + TOD - 2050 | 6,445,144 | 894,767 | 7.2 | 66% | 2,386,408 | 18,971,187 | 553,042 | 634 |

It is observed that the average speed shall increase from 5.5 kph and 1.45 kph in BAU to   10.8 Kph & 7.2 Kph respectively in 2030 & 2050. The share of public transport (bus, metro, IPT) increases from 45% in 2030 and 39% in 2050 to 58% and 66% respectively. There is a decrease in Co2 emissions observed from 946 tonnes to 606 tonnes in 2030 and from 508 tonnes to 634 tonnes in 2050 respectively.

Police Bundle 2: Travel Demand Management

Policy measure : Increase in Fuel Cost

Background

In this scenario, increase in fuel cost has been considered to discourage use of personalised vehicles  which will also directly reduce the emissions under the transportation sector and also increase the sustainable modes share  within the city. The details of this scenario is explained in detail in the below sections.

Scenario planning inputs

For this scenario, past trend of increase in petrol, diesel and CNG price were studied and estimated for the year 2030 and 2050 (Fig 2.10, Fig 2.11, Fig 2.12).  To the estimated fuel cost, an additional cost has considered to encourage use of public transport and sustainable modes.

Figure 2.10. : Past trend of petrol price

Figure 2.11: Past Trend of Diesel Price

Figure 2.12: Past trend of CNG Price

Table 2.10 and Table 2.11 shows increased fuel cost consumption assumption and mode wise fuel consumption cost in 2030 and 2050

Table 2.10: Increased Fuel cost assumptions

| Year | Increase Every year |
| --- | --- |
| 2020 - 2030 | 1.50% |
| 2030 -2040 | 1.25% |
| 2040 - 2050 | 1.00% |

Table 2.11: Increased fuel cost of various available modes for 2030 & 2050

| 2030 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Mode | Fuel Price | Mileage (in KM) | Per KM Cost | Increase In Fuel Price |
| 2 Wheeler | 111 | 45 | 2.48 | 2.92 |
| Car | 102 | 16 | 6.39 | 7.54 |
| Taxi | 102 | 16 | 6.39 | 7.54 |
| Auto | 51 | 22 | 2.31 | 2.73 |
| Shared Auto | 51 | 22 | 2.31 | 2.73 |

Modal Split

Base year calibrated parameters are used to develop and estimate the future year 2030 and 2050 network. The estimate shows that there is a change in public transport from base year 2011 (50%) to 2030 (52%) with a positive shift of +2%. For the horizon year 2050, public transport share is increase +3% with comparing to 2030 as the additional development is considered along the mass transit system & congestion on road increases. Table 2.12 and Table 2.13 below shows the mode wise percentage share of trips in the cardinal years 2030 and 2050 and comparison between BAU and alternate scenario

Table 2.12 Comparison of mode split ( home based and non home based) in BAU and Increase in Fuel cost -2030 & 2050 (Per day)

| Mode |  | BAU - 2030 | % | Trips - 2030 | % | BAU - 2050 | % | Trips - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | HB | 3754502 | 23% | 2480750 | 15% | 5523972 | 26% | 3138997 | 15% |
| Put |  | 5223359 | 32% | 7791413 | 48% | 5059888 | 24% | 10647694 | 50% |
| Auto |  | 367309 | 2.2% | 261536 | 1.6% | 460525 | 2.1% | 313609 | 1% |
| Tw |  | 7055831 | 43% | 5867302 | 36% | 10438613 | 49% | 7382698 | 34% |
| Total |  | 16401001 | 100% | 16401001 | 100% | 21482998 | 100% | 21482998 | 100% |
| Car | NHB | 4092253 | 21% | 3821919 | 19% | 5896294 | 23% | 4616088 | 18% |
| Put |  | 10830512 | 55% | 10829198 | 55% | 13452639 | 52% | 15339046 | 60% |
| Auto |  | 2069007 | 11% | 1942594 | 10% | 2760262 | 11% | 2258239 | 9% |
| Tw |  | 2639729 | 13% | 3037791 | 15% | 3605302 | 14% | 3501125 | 14% |
| Total |  | 19631502 | 100% | 19631502 | 100% | 25714498 | 100% | 25714498 | 100% |

Table 2.13: Comparison of mode split in BAU and Increase in Fuel cost -2030 & 2050 (per day)

| Mode | BAU - 2030 | % | Trips - 2030 | % | BAU - 2050 | % | Trips - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | 7846755 | 22% | 6302669 | 17% | 11420266 | 24% | 7755085 | 16% |
| Put | 16053871 | 45% | 18620611 | 52% | 18512527 | 39% | 25986740 | 55% |
| Auto | 2436316 | 7% | 2204130 | 6% | 3220787 | 7% | 2571848 | 5% |
| Tw | 9695560 | 27% | 8905093 | 25% | 14043915 | 30% | 10883823 | 23% |
| Total | 36032503 | 100% | 36032503 | 100% | 47197496 | 100% | 47197496 | 100% |

Trip Assignment

The horizon year trips for 2030 and 2050 were assigned on the proposed transport network and congested links were identified as shown in the figure 2.13 and fig 2.14  below:

Figure 2-13 Trip Assignment for the horizon year - 2030

Figure 2-14 Trip Assignment for the horizon year – 2050

The comparative evaluation of alternative networks has been carried out from the point of view of passenger transport demand for the horizon year and ability of the various networks to cater to this demand. Table  2.14  shows the comparative evaluation of alternate scenarios

Table 2.14: Comparative evaluation of Base year , BAU & Increase in Fuel cost - 2030 & 2050 scenario (Peak hour)

| Transport Network Scenario | Veh Dist Travelled (in Km) | Veh Travelled Hours | Average Speed (Kmph) | % PT+IPT Share | Pax (in No's) | Pax Travelled Km | Pax Travelled Hours | Co2 (in Kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base Year | 2,465,650 | 165,583 | 14.9 | 50% | 434,197 | 3,426,571 | 101,350 | 230 |
| BAU - 2030 | 11,316,713 | 2,064,762 | 5.5 | 45% | 1,213,903 | 10,911,894 | 334,778 | 946 |
| BAU - 2050 | 19,821,085 | 13,671,726 | 1.4 | 39% | 1,385,215 | 12,974,539 | 402,913 | 1642 |
| Increase in Fuel - 2030 | 8,354,030 | 1,176,685 | 7.1 | 52% | 1,491,981 | 13,522,576 | 412,764 | 698 |
| Increase in Fuel - 2050 | 11,288,483 | 7,676,688 | 1.5 | 55% | 2,105,999 | 19,617,760 | 1,509,153 | 966 |

It is observed that the average speed shall increase from 5.5 kph and 1.45 kph in BAU to   7.1 Kph & 1.5 Kph respectively in 2030 & 2050. The share of public transport (bus, metro, IPT) increases from 45% in 2030 and 39% in 2050 to 52% and 55% respectively. There is a decrease in Co2 emissions observed from 946 tonnes to 698 tonnes in 2030 and from 1642 tonnes to 966 tonnes in 2050 respectively.

Policy measure : Congestion Pricing

Background

In this scenario, congestion pricing has been considered to encourage the public transport which will also directly reduce the emissions under the transportation sector and also increase the sustainable modes share within the city. The details of this scenario is explained in detail in the below sections.

Scenario planning inputs

For this scenario, congestion pricing cost per hour was adopted from pervious related studies, for this study Rajeev chowk and its radial roads considered and estimated for the year 2030 and 2050. To the estimated congestion pricing cost, an additional cost has considered to encourage the public transport and the sustainable modes.

Deriving Marginal cost

Marginal congestion cost  = q(b/V^2)(dv/dq)

Where,

dv/dq – the slope of the speed/ flow relationship, which varies with the type of road and volume of traffic

q – the volume of traffic (in Pcu’s)

v – the resulting speed, which varies with type of road and volume of traffic

b – The value of time, which varies with the mix of journey purpose and income of the users

Marginal congestion cost is 205/- Per Hour (Source : CEPT Master’s Thesis 2016)

Modal Split

Base year calibrated parameters are used to develop and estimate the future year 2030 and 2050 network. The estimate shows that there is a change in public transport from base year 2011 (50%) to 2030 (54%) with a positive shift of +4%. For the horizon year 2050, public transport share is increase +3% with comparing to 2030 as the additional development is considered along the mass transit system & congestion on road increases. Table 2.15 below shows the mode wise percentage share of trips in the cardinal years 2030 and 2050 and Table 2.16 shows comparison between BAU and alternate policy scenario

Table 2-15 Comparison of mode split in BAU & Congestion Pricing - 2030 & 2050 (per day)

| Mode |  | BAU - 2030 | % | Trips - 2030 | % | BAU - 2050 | % | Trips - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | HW | 3754502 | 23% | 2659504 | 16% | 5523972 | 26% | 3372194 | 16% |
| Put |  | 5223359 | 32% | 7055478 | 43% | 5059888 | 24% | 9693119 | 45% |
| Auto |  | 367309 | 2.2% | 268054 | 1.6% | 460525 | 2.1% | 321607 | 1% |
| Tw |  | 7055831 | 43% | 6417964 | 39% | 10438613 | 49% | 8096078 | 38% |
| Total |  | 16401001 | 100% | 16401001 | 100% | 21482998 | 100% | 21482998 | 100% |
| Car | NHW | 4092253 | 21% | 3091899 | 16% | 5896294 | 23% | 3702547 | 14% |
| Put |  | 10830512 | 55% | 12466652 | 64% | 13452639 | 52% | 17318448 | 67% |
| Auto |  | 2069007 | 11% | 1643759 | 8% | 2760262 | 11% | 1897495 | 7% |
| Tw |  | 2639729 | 13% | 2429192 | 12% | 3605302 | 14% | 2796007 | 11% |
| Total |  | 19631502 | 100% | 19631502 | 100% | 25714498 | 100% | 25714498 | 100% |

Table 2-16: Comparison of mode split in BAU & Congestion Pricing - 2030 & 2050 (per day)

| Mode | BAU - 2030 | % | Trips - 2030 | % | BAU - 2050 | % | Trips - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | 7846755 | 22% | 5751403 | 16% | 11420266 | 24% | 7074741 | 15% |
| Put | 16053871 | 45% | 19522131 | 54% | 18512527 | 39% | 27011567 | 57% |
| Auto | 2436316 | 7% | 1911813 | 5% | 3220787 | 7% | 2219103 | 5% |
| Tw | 9695560 | 27% | 8847156 | 25% | 14043915 | 30% | 10892085 | 23% |
| Total | 36032503 | 100% | 36032503 | 100% | 47197496 | 100% | 47197496 | 100% |

Trip Assignment

The horizon year trips for 2030 and 2050 were assigned on the proposed transport network and congested links were identified as shown in figure 2.15 and 2.16 below:

Figure 2.15 Trip Assignment for the horizon year - 2030

Figure 2.16 Trip Assignment for the horizon year – 2050

The comparative evaluation of alternative networks has been carried out from the point of view of passenger transport demand for the horizon year and ability of the various networks to cater to this demand.. Table  2.17  shows the comparative evaluation of alternate scenarios

Table 2-17: Comparison of Base year, BAU & Congestion pricing - 2030 & 2050 scenario (peak hour)

| Transport Network Scenario | Veh Dist Travelled (in Km) | Veh Travelled Hours | Average Speed (Kmph) | % PT+IPT Share | Pax (in No's) | Pax Travelled Km | Pax Travelled Hours | Co2 (in Kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base Year | 2,465,650 | 165,583 | 14.9 | 50% | 434,197 | 3,426,571 | 101,350 | 230 |
| BAU - 2030 | 11,316,713 | 2,064,762 | 5.5 | 45% | 1,213,903 | 10,911,894 | 334,778 | 946 |
| BAU - 2050 | 19,821,085 | 13,671,726 | 1.4 | 39% | 1,385,215 | 12,974,539 | 402,913 | 1642 |
| Congestion - 2030 | 7,909,869 | 1,089,366 | 7.3 | 54% | 1,497,609 | 13,182,398 | 403,294 | 651 |
| Congestion - 2050 | 10,760,649 | 3,482,328 | 3.1 | 57% | 2,104,625 | 19,145,290 | 590,319 | 906 |

It is observed that the average speed shall increase from 5.5 kph and 1.45 kph in BAU to   7.3 Kph & 3.1 Kph respectively in 2030 & 2050. The share of public transport (bus, metro, IPT) increases from 45% in 2030 and 39% in 2050 to 54% and 57% respectively. There is a decrease in Co2 emissions observed from 946 tonnes to 651 tonnes in 2030 and from 1642 tonnes to 906 tonnes in 2050 respectively.

Policy measure : Combination of Fuel pricing + Congestion pricing scenarios

Modal Split

Base year calibrated parameters are used to develop and estimate the future year 2030 and 2050 network. The estimate shows that there is a change in public transport from base year 2011 (50%) to 2030 (57%) with a positive shift of +7%. For the horizon year 2050, public transport share is increase +4% with comparing to 2030 as the additional development is considered along the mass transit system & congestion on road increases. Table 2.18 and Table 2.19 below shows the mode wise percentage share of trips in the cardinal years 2030 and 2050.

Table 2.18 Comparison of mode split  (home based and non home based) in Base year and BAU and Fuel pricing + Congestion Pricing -2030 & 2050 (per day)

| Mode |  | BAU - 2030 | % | Trips - 2030 | % | BAU - 2050 | % | Trips - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | HB | 3754502 | 23% | 2242110 | 14% | 5523972 | 26% | 2755342 | 13% |
| Put |  | 5223359 | 32% | 8751282 | 53% | 5059888 | 24% | 12172628 | 57% |
| Auto |  | 367309 | 2.2% | 251510 | 1.5% | 460525 | 2.1% | 298008 | 1% |
| Tw |  | 7055831 | 43% | 5156099 | 31% | 10438613 | 49% | 6257020 | 29% |
| Total |  | 16401001 | 100% | 16401001 | 100% | 21482998 | 100% | 21482998 | 100% |
| Car | NHB | 4092253 | 21% | 3404212 | 17% | 5896294 | 23% | 3957499 | 15% |
| Put |  | 10830512 | 55% | 11767269 | 60% | 13452639 | 52% | 16763770 | 65% |
| Auto |  | 2069007 | 11% | 1772232 | 9% | 2760262 | 11% | 1999055 | 8% |
| Tw |  | 2639729 | 13% | 2687789 | 14% | 3605302 | 14% | 2994173 | 12% |
| Total |  | 19631502 | 100% | 19631502 | 100% | 25714498 | 100% | 25714498 | 100% |

Table 2-19: Comparison of mode split in Base year and BAU & Fuel pricing + congestion pricing -2030 & 2050 (per day)

| Mode | BAU - 2030 | % | Trips - 2030 | % | BAU - 2050 | % | Trips - 2050 | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | 7846755 | 22% | 5646322 | 16% | 11420266 | 24% | 6712841 | 14% |
| Put | 16053871 | 45% | 20518551 | 57% | 18512527 | 39% | 28936399 | 61% |
| Auto | 2436316 | 7% | 2023742 | 6% | 3220787 | 7% | 2297063 | 5% |
| Tw | 9695560 | 27% | 7843888 | 22% | 14043915 | 30% | 9251193 | 20% |
| Total | 36032503 | 100% | 36032503 | 100% | 47197496 | 100% | 47197496 | 100% |

Trip Assignment

The horizon year trips for 2030 and 2050 were assigned on the proposed transport network and congested links were identified as shown in the figure 2.17 below:

Figure 2-17 Trip Assignment for the horizon year – 2050

The comparative evaluation of alternative networks has been carried out from the point of view of passenger transport demand for the horizon year and ability of the various networks to cater to this demand.. Table  2.20  shows the comparative evaluation of alternate scenarios

Table 2-20: Comparison of Base year and BAU and Fuel pricing + Congestion pricing - 2030 & 2050 scenario (peak hour)

| Transport Network Scenario | Veh Dist Travelled (in Km) | Veh Travelled Hours | Average Speed (Kmph) | % PT+IPT Share | Pax (in No's) | Pax Travelled Km | Pax Travelled Hours | Co2 (in Kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base Year | 2,465,650 | 165,583 | 14.9 | 50% | 434,197 | 3,426,571 | 101,350 | 230 |
| BAU - 2030 | 11,316,713 | 2,064,762 | 5 | 45% | 1,213,903 | 10,911,894 | 334,778 | 946 |
| BAU - 2050 | 19,821,085 | 13,671,726 | 1.4 | 39% | 1,385,215 | 12,974,539 | 402,913 | 1642 |
| Congestion + Fuel pricing- 2030 | 7,102,901 | 743,951 | 9.5 | 57% | 1,642,382 | 14,621,053 | 445,788 | 618 |
| Congestion + Fuel pricing - 2050 | 9,097,963 | 1,905,705 | 4.8 | 61% | 2,343,329 | 21,370,297 | 656,640 | 821 |

It is observed that the average speed shall increase from 5.5 kph and 1.45 kph in BAU to   9.5 Kph & 4.8 Kph respectively in 2030 & 2050. The share of public transport (bus, metro, IPT) increases from 45% in 2030 and 39% in 2050 to 57% and 61% respectively. There is a decrease in Co2 emissions observed from 946 tonnes to 618 tonnes in 2030 and from 1642 tonnes to 821 tonnes in 2050 respectively.

Policy measure : Combination of Car Pooling and Car Sharing + Fuel Price + Congestion Pricing

Background

In this scenario, congestion pricing has been considered to encourage the public transport which will also directly reduce the emissions under the transportation sector and also increase the sustainable modes share within the city. The details of this scenario is explained in detail in the below sections.

Scenario - planning inputs

For this scenario, growth in app based (aggregate cabs) taxi (Ola/ Uber) is considered and car-pooling strategy has been adopted and estimated for 2030 and 2050.

Growth in App based cabs

Form the secondary data in Delhi it is observed that there is tremendous growth in app based car users with a CAGR of 8.84%. The table below shows the growth of app based cars (Ola/ Uber).

| Ola / Uber Cars | Units | Passenger / day | Total Demand |
| --- | --- | --- | --- |
| FY 2013 | 150000 | 19 | 2850000 |
| FY 2016 | 245000 | 15 | 3675000 |
| CAGR (2016 - 2013) | 17.8% |  | 8.84% |

The estimated demand of app based cab users for 2030 and 2050 is shown below:

| HY | GR% | Demand of app Based Users |
| --- | --- | --- |
| 2020 | 7.0% | 47.78 |
| 2025 | 5.5% | 59.72 |
| 2030 | 4.5% | 71.66 |
| 2035 | 3.5% | 82.41 |
| 2040 | 2.5% | 90.65 |
| 2045 | 1.5% | 95.19 |
| 2050 | 1.0% | 97.57 |

Estimated shift of users towards app based cabs from other modes

SPA(2017) conducted an in -house study in 2017 for this project  to understand the mode choice of users with giving taxi as an option. The percentages shift from different modes have been suitably adopted for 2030 and 2050 respectively. Table below shows the percentage shift towards taxi from available modes.

| Shift Towards Hire Taxi | 2030 | 2050 |
| --- | --- | --- |
| Car | 30.0% | 40.0% |
| Auto | 50.0% | 60.0% |
| TW | 50.0% | 60.0% |

Modal Split

Base year calibrated parameters are used to develop and estimate the future year 2030 and 2050 network. The estimate shows that there is a change in public transport from base year 2011 (50%) to 2030 (45%) with a negative shift of -5%. For the horizon year 2050, public transport share is likely to  decrease by 5%  comparing to 2030  owing to  increase in app based car users .Table 2.21 shows the mode wise percentage share of trips in the cardinal years 2030 and 2050.

Table 2-21 Comparison of mode split of Car sharing + Fuel cost + Congestion pricing -2030 & 2050 (per day)

| Mode | 2030 | % Share | 2050 | % Share |
| --- | --- | --- | --- | --- |
| Car | 3952425 | 11.0% | 4027705 | 8.5% |
| PuT | 20518551 | 56.9% | 28936399 | 61.3% |
| Auto | 1011871 | 2.8% | 918825 | 1.9% |
| Tw | 3921944 | 10.9% | 3700477 | 7.8% |
| Taxi | 6627712 | 18.4% | 9614090 | 20.4% |
| Total | 36032503 | 100.0% | 47197496 | 100.0% |

Assumptions on Occupancy rate

It is assumed that the occupancy rates of cars and cabs as a result of policy of government to enforce car pooling will increase by 25% by the year of 2050

| Occupancy Rate | Base Year | 2030 | 2050 |
| --- | --- | --- | --- |
| Car + Taxi | 2.2 | 2.5 | 2.8 |

Trip Assignment

The horizon year trips for 2030 and 2050 were assigned on the proposed transport network and congested links were identified as shown in the figure 2.18 below:

Figure 2-18 Trip Assignment for the horizon year – 2050

The comparative evaluation of alternative networks has been carried out from the point of view of passenger transport demand for the horizon year and ability of the various networks to cater to this demand.. Table  2.22  shows the comparative evaluation of alternate scenarios

Table 2-22: Comparison of Base year and BAU & Car sharing + Fuel cost + Congestion pricing -2030 & 2050 (Peak hour)

| Transport Network Scenario | Veh Dist Travelled (in Km) | Veh Travelled Hours | Average Speed (Kmph) | % PT+IPT Share | Pax (in No's) | Pax Travelled Km | Pax Travelled Hours | Co2 (in Kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base Year | 2,465,650 | 165,583 | 14.9 | 50% | 434,197 | 3,426,571 | 101,350 | 230 |
| BAU - 2030 | 11,316,713 | 2,064,762 | 5.5 | 45% | 1,213,903 | 10,911,894 | 334,778 | 946 |
| BAU - 2050 | 19,821,085 | 13,671,726 | 1.4 | 39% | 1,385,215 | 12,974,539 | 402,913 | 1642 |
| Congestion + Fuel + Car Share - 2030 | 6,272,909 | 981,306 | 6.4 | 57% | 1,642,382 | 14,621,053 | 445,788 | 748 |
| Congestion + Fuel + Car 2050 | 7,834,893 | 1,707,368 | 4.6 | 61% | 2,343,329 | 21,370,297 | 656,640 | 964 |

It is observed that the average speed shall increase from 5.5 kph and 1.45 kph in BAU to   6.4 Kph & 4.6 Kph respectively in 2030 & 2050. The share of public transport (bus, metro, IPT) increases from 45% in 2030 and 39% in 2050 to 57% and 61% respectively. There is a decrease in Co2 emissions observed from 946 tonnes to 748 tonnes in 2030 and from 1642 tonnes to 964 tonnes in 2050 respectively.

Police Bundle 3: Technology adaptation

Policy measure : Electric Mobility

Background

Electric mobility is on top agenda of the government in India as outlined by a policy paper of the Niti Ayog published recently . Accordingly under the technology adaptation scenario, electric mobility has been considered  which shall aim to reduce the emissions under the transportation sector and also increase the sustainable modes share within the city. The details of this scenario is explained in detail in the below sections.

Scenario - planning inputs

Rapidly evolving technologies and business models for delivering mobility services have dramatic potential to transform the global transportation sector in the years ahead. New and fundamentally different pathways are emerging to provide clean, cost-effective mobility services, creating new jobs, reducing oil import dependence, achieving more efficient land use in cities, and improving public health. India is uniquely positioned to take advantage of these developments due to a set of advantageous conditions and capabilities.

Forecast for Electric vehicle in India

Niti Aayog (previously Planning Commission) has forecasted the target for electric mobility share in India in the transformative scenario for the year 2030. Table 2.23 below shows the estimated share of electric mobility scenario by the year 2030.

Table 2-23 : Forecast of Electric Mobility in India - 2030

| Category | 2015 All scenario | 2030 |  |
| --- | --- | --- | --- |
|  |  | BAU (%) | Transformative (%) |
| Ownership Mix of 4 wheel |  |  |  |
| Private | 73 | 77 | 50 |
| Commercial | 27 | 23 | 50 |
| Percent Electric |  |  |  |
| 2 Wheel | 0 | 5 | 40 |
| 3 Wheel | 0 | 5 | 100 |
| 4Wheel |  |  |  |
| Personal | 0 | 1 | 40 BEV |
| Commercial | 0 | 5 | 100 BEV |
| Public Transit | 0 | 1 | 100 |

Adopted Electric mobility Share in Delhi

The targets of Nitti Aayog in its policy paper appears  at the higher side. For this study keeping in view the existing status of electric mobility in India and capacity limitations for leap frogging to attain the desired targets it is assumed that 50% of Niti Aayog targets shall be achieved in year 2030 while full targets shall be achieved by 2050. Table 2.24 below shows the adopted targets of share of electric mobility vehicle wise in Delhi

Table 2-24 : Adopted Electric mobility share in Delhi by 2030 & 2050

| Electric vehicles | % share in 2030 | % share in 2050 |
| --- | --- | --- |
| Two Wheeler | 20% | 40% |
| Auto (Three wheeler) | 50% | 100% |
| Personal Car | 20% | 40% |
| Taxi (Commercial) | 50% | 100% |
| Public Transport | 50% | 100% |

BAU scenario with Electric Mobility

Based on the above assumption vehicle travelled Kilometres were further classified into electrical and non- electrical vehicles and these values were used to estimate the emissions by the year 2030 and 2050. Table 2.25, Table 2.26 and Table 2.27 below shows the travelled vehicle kilometre and emissions by 2030 and 2050 respectively.

Table 2-25 : BAU with Electric Mobility - Vehicle travelled KM by 2030 & 2050 (peak hour)

| Mode (Travelled Veh. KM) | 2030 - Normal | 2030 - Electrical | 2050 - Normal | 2050 - Electrical |
| --- | --- | --- | --- | --- |
| Car | 2892504 | 723126 | 3849746 | 2566497 |
| Auto | 245187 | 245187 | 0 | 780523 |
| Tw | 5768567 | 1442142 | 7574592 | 5049728 |
| Bus | 3380681 | 3380681 | 0 | 8389044 |
| Metro |  | 4150532 |  | 4585495 |

Table 2-26 : BAU with Electric Mobility –Emissions by 2030 (peak hour)

| 2030 | Emissions (in Kg) & Only Co2 in Tones |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Modes/ Emissions | CO | HC | NOx | CO2 | PM |
| Car | 1527 | 385 | 377 | 417 | 12 |
| Auto | 119 | 77 | 36 | 20 | 4 |
| Tw | 5382 | 1159 | 329 | 246 | 63 |
| Bus | 112 | 29 | 52 | 38 | 1 |
| Metro | 0 | 0 | 0 | 0 | 0 |
| Total | 7140 | 1651 | 794 | 722 | 80 |

Table 2-27 : BAU with Electric Mobility –Emissions by 2050 (peak hour)

| 2050 | Emissions (in Kg) & Only Co2 in Tones |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Modes/ Emissions | CO | HC | NOx | CO2 | PM |
| Car | 2033 | 512 | 77 | 567 | 15 |
| Auto | 0 | 0 | 0 | 0 | 0 |
| Tw | 7529 | 750 | 454 | 330 | 30 |
| Bus | 0 | 0 | 0 | 0 | 0 |
| Metro | 0 | 0 | 0 | 0 | 0 |
| Total | 9562 | 1262 | 532 | 897 | 45 |

Combination of Policy Bundle 1: Planning Policies with Electric Mobility

Based on the above assumption vehicle travelled Kilometres were further classified into electrical and non- electrical vehicles and these values were used to estimate the emissions by the year 2030 and 2050. To have a greater impact, electric mobility scenario was added with planning combine police scenario to understand impact of these two police bundle combination. Table 2.28, Table 2.29 and Table 2.30 below shows the classified  travelled vehicle kilometre and emissions by 2030 and 2050 respectively

Table 2-28 : Policy Bundle: Planning with Electric Mobility - Vehicle travelled KM by 2030 & 2050 (peak hour)

| Mode (Travelled Veh. KM) | 2030 - Normal | 2030 - Electrical | 2050 - Normal | 2050 - Electrical |
| --- | --- | --- | --- | --- |
| Car | 1723634 | 430908 | 1346627 | 897751 |
| Auto | 187642 | 187642 | 0 | 471753 |
| Tw | 3068446 | 767111 | 2237408 | 1491605 |
| Bus | 107606 | 107606 | 0 | 9633130 |
| Metro |  | 12097927 |  | 9338058 |

Table 2-29 : Policy Bundle: Planning with Electric Mobility -  Emissions by 2030 (peak hour)

| 2030 | Emissions (in Kg) & Only Co2 in Tones |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Modes/ Emissions | CO | HC | NOx | CO2 | PM |
| Car | 910 | 229 | 225 | 249 | 7 |
| Auto | 91 | 59 | 28 | 15 | 3 |
| Tw | 2863 | 617 | 175 | 131 | 34 |
| Bus | 4 | 1 | 2 | 1 | 0 |
| Metro | 0 | 0 | 0 | 0 | 0 |
| Total | 3868 | 906 | 429 | 396 | 44 |

Table 2-30 : Policy Bundle: Planning with Electric Mobility – Emissions by 2050 (peak hour)

| 2050 | Emissions (in Kg) & Only Co2 in Tones |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Modes/ Emissions | CO | HC | NOx | CO2 | PM |
| Car | 711 | 179 | 27 | 198 | 5 |
| Auto | 0 | 0 | 0 | 0 | 0 |
| Tw | 2224 | 222 | 134 | 98 | 9 |
| Bus | 0 | 0 | 0 | 0 | 0 |
| Metro | 0 | 0 | 0 | 0 | 0 |
| Total | 2935 | 401 | 162 | 296 | 14 |

Policy Bundle 2: Travel Demand Management with Electric Mobility

Based on the above assumption vehicle travelled Kilometres were further classified into electrical and non- electrical vehicles and these values were used to estimate the emissions by the year 2030 and 2050. To have a greater impact, electric mobility scenario was added with travel demand management combined police scenario to understand impact of these two police bundle combination. Table 2.31 , Table 3.2 and Table 3.3 below shows the mode wise travelled vehicle kilometre and emissions by 2030 and 2050 respectively.

Table 2-31 : Policy Bundle: Travel Demand Management with Electric Mobility - Vehicle travelled KM by 2030 & 2050 (peak hour)

| Mode (Travelled Veh. KM) | 2030 - Normal | 2030 - Electrical | 2050 - Normal | 2050 - Electrical |
| --- | --- | --- | --- | --- |
| Car | 2924702 | 731175 | 2698461 | 1798974 |
| Auto | 101387 | 101387 | 0 | 213658 |
| Tw | 1931406 | 482852 | 1874279 | 1249520 |
| Bus | 4452091 | 4452091 | 0 | 13411972 |
| Metro |  | 5716872 |  | 7958324 |

Table 2-32 : Police Bundle: Travel Demand Management with Electric Mobility – emissions by 2030 (peak hour)

| 2030 | Emissions (in Kg) & Only Co2 in Tonnes |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Modes/ Emissions | CO | HC | NOx | CO2 | PM |
| Car | 1544 | 389 | 381 | 422 | 12 |
| Auto | 49 | 32 | 15 | 8 | 2 |
| Tw | 1802 | 388 | 110 | 82 | 21 |
| Bus | 147 | 38 | 68 | 50 | 1 |
| Metro | 0 | 0 | 0 | 0 | 0 |
| Total | 3543 | 847 | 574 | 563 | 36 |

Table 2-33 : Police Bundle: Travel Demand Management with Electric Mobility – emissions by 2050 (peak hour)

| 2050 | Emissions (in Kg) & Only Co2 in Tonnes |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Modes/ Emissions | CO | HC | NOx | CO2 | PM |
| Car | 1425 | 359 | 54 | 397 | 10 |
| Auto | 0 | 0 | 0 | 0 | 0 |
| Tw | 55 | 17 | 23 | 21 | 0 |
| Bus | 0 | 0 | 0 | 0 | 0 |
| Metro | 0 | 0 | 0 | 0 | 0 |
| Total | 1480 | 376 | 77 | 418 | 11 |

Source : Emissions factors were adopted from IIT-Mumbai (Refer Annexure – 4)

- Comparison of BAU & Policy Scenarios

Comparison of BAU & Policy scenarios (Transport parameters)

Scenarios were evaluated based on the identified network parameters such as peak hour speed (Kmph) and percentage of public transport share. Table 3.1 below shows the comparison of all adopted polices scenarios and BAU to understand the benefits of each policy scenario and there combinations together.

Comparison of BAU & Policy scenario (GHG Emissions)

Table 3.2 shows comparative evaluation of BAU and alternate policy scenarios

Table 3.2 Comparative Evaluation of BAU and Alternate policy scenarios (Peak hour)

| Police Bundle | Transport Network Scenario | CO (Kg) | HC (Kg) | Nox (Kg) | CO2 (Tones) | PM (Kg) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Base Year | 7361 | 2667 | 1107 | 230 | 250 |
| BAU | BAU - 2030 | 9099 | 2143 | 1057 | 946 | 104 |
|  | BAU - 2050 | 16420 | 2340 | 1052 | 1642 | 85 |
| Policy Bundle 1 : Planning | Increase in Transit - 2030 | 6943 | 1641 | 839 | 742 | 80 |
|  | Increase in Transit - 2050 | 8464 | 1247 | 620 | 902 | 45 |
|  | TOD - 2030 | 5582 | 1338 | 687 | 606 | 65 |
|  | TOD - 2050 | 6422 | 985 | 490 | 726 | 35 |
|  | Transit + TOD - 2030 | 4906 | 1178 | 558 | 508 | 57 |
|  | Transit + TOD - 2050 | 5319 | 853 | 424 | 634 | 31 |
| Policy Bundle 2 : Travel Demand Management | Increase in Fuel - 2030 | 6960 | 1640 | 806 | 698 | 81 |
|  | Increase in Fuel - 2050 | 9787 | 1390 | 714 | 966 | 51 |
|  | Congestion - 2030 | 6676 | 1554 | 753 | 651 | 77 |
|  | Congestion - 2050 | 9481 | 1312 | 689 | 906 | 48 |
|  | Congestion + Fuel - 2030 | 5966 | 1411 | 717 | 618 | 69 |
|  | Congestion + Fuel - 2050 | 7967 | 1154 | 616 | 821 | 42 |
|  | Congestion + Fuel + Car Share - 2030 | 4576 | 1112 | 779 | 748 | 47 |
|  | Congestion + Fuel + Car Share - 2050 | 5941 | 1074 | 457 | 964 | 35 |
| Policy Bundle 3: Technology adaptation | BAU with Electric Mobility - 2030 | 7140 | 1651 | 794 | 722 | 80 |
|  | BAU with Electric Mobility - 2050 | 9562 | 1262 | 532 | 897 | 45 |
| Combination of policy Bundles | Planning Policy with Electric Mobility 2030 | 3868 | 906 | 429 | 396 | 44 |
|  | Planning Policy with Electric Mobility 2050 | 2935 | 401 | 162 | 296 | 14 |
|  | TDM with Electric Mobility 2030 | 3543 | 847 | 574 | 563 | 36 |
|  | TDM with Electric Mobility 2050 | 1480 | 376 | 77 | 418 | 11 |

Fig 3.1 to 3.6 graphically various impacts of various policy bundles on emissions in peak hour

Figure 3-1: Comparison of Co2 Emissions of various scenarios

Figure 3-2 : Emissions comparison of Base year & BAU Scenario

Figure 3-3 : Emissions comparison of Policy Bundle 1: Planning

Figure 3-4 : Emissions comparison of Policy Bundle 2: Travel demand Management (TDM)

Figure 3-5 : Emissions comparison of Policy Bundle 3: Electric Mobility

Figure 3-6 : Emissions comparison of Policy Bundle 1 & 2 with Electric Mobility

Table 3-1 : Comparison of BAU & Policy scenario (Transport parameters) – 2030 & 2050 (peak hour)

| Police Bundle | Transport Network Scenario | Veh Dist Travelled (in Km) | Veh Travelled Hours | Average Speed (Kmph) | % PT+IPT Share | Pax (in No's) | Pax Travelled Km | Pax Travelled Hours | Co2 (in tons) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Base Year | 2,465,650 | 165,583 | 14.9 | 50% | 434,197 | 3,426,571 | 101,350 | 230 |
| BAU | BAU - 2030 | 11,316,713 | 2,064,762 | 5.5 | 45% | 1,213,903 | 10,911,894 | 334,778 | 946 |
|  | BAU - 2050 | 19,821,085 | 13,671,726 | 1.4 | 39% | 1,385,215 | 12,974,539 | 402,913 | 1,642 |
| Police Bundle 1 : Planning | Increase in Transit - 2030 | 8,508,953 | 1,036,666 | 8.2 | 54% | 1,497,529 | 13,321,070 | 402,068 | 742 |
|  | Increase in Transit - 2050 | 9,973,066 | 2,067,928 | 4.8 | 61% | 2,311,025 | 20,808,769 | 616,009 | 902 |
|  | Transit Oriented development - 2030 | 6,905,254 | 653,359 | 10.6 | 56% | 1,500,347 | 11,831,675 | 352,106 | 606 |
|  | Transit Oriented development - 2050 | 7,664,055 | 1,107,692 | 6.9 | 62% | 2,243,830 | 17,980,208 | 542,521 | 726 |
|  | Transit + TOD - 2030 | 6,365,383 | 589,818 | 10.8 | 58% | 1,546,676 | 12,313,139 | 364,058 | 508 |
|  | Transit + TOD - 2050 | 6,445,144 | 894,767 | 7.2 | 66% | 2,386,408 | 18,971,187 | 553,042 | 634 |
| Police Bundle 2 : Travel Demand Management | Increase in Fuel cost- 2030 | 8,354,030 | 1,176,685 | 7.1 | 52% | 1,491,981 | 13,522,576 | 412,764 | 698 |
|  | Increase in Fuel cost - 2050 | 11,288,483 | 7,676,688 | 1.5 | 55% | 2,105,999 | 19,617,760 | 1,509,153 | 966 |
|  | Congestion Pricing - 2030 | 7,909,869 | 1,089,366 | 7.3 | 54% | 1,497,609 | 13,182,398 | 403,294 | 651 |
|  | Congestion Pricing - 2050 | 10,760,649 | 3,482,328 | 3.1 | 57% | 2,104,625 | 19,145,290 | 590,319 | 906 |
|  | Congestion Pricing + Fuel cost- 2030 | 7,102,901 | 743,951 | 9.5 | 57% | 1,642,382 | 14,621,053 | 445,788 | 618 |
|  | Congestion Pricing + Fuel cost- 2050 | 9,097,963 | 1,905,705 | 4.8 | 61% | 2,343,329 | 21,370,297 | 656,640 | 821 |
|  | Congestion Pricing+ Fuel cost+ Car Share - 2030 | 6,272,909 | 981,306 | 6.4 | 57% | 1,642,382 | 14,621,053 | 445,788 | 748 |
|  | Congestion Pricing+ Fuel cost+ Car Share - 2050 | 7,834,893 | 1,707,368 | 4.6 | 61% | 2,343,329 | 21,370,297 | 656,640 | 964 |

Annexure

| Emission Factors for Cars |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | gm/km |  |  |  |  |
|  | CO | HC | NOx | CO2 | PM |
| 2005 | 2.444 | 0.267 | 0.344 | 133.576 | 0.0276 |
| 2021 | 0.604 | 0.139 | 0.178 | 144.126 | 0.004 |
| 2031 | 0.528 | 0.133 | 0.13 | 144.29 | 0.004 |
| 2050 | 0.528 | 0.133 | 0.020 | 147.116 | 0.0038 |
| Emission Factors for Two Wheelers |  |  |  |  |  |
|  | gm/km |  |  |  |  |
|  | CO | HC | NOx | CO2 | PM |
| 2005 | 1.153 | 0.790 | 0.112 | 31.065 | 0.023 |
| 2021 | 0.628 | 0.416 | 0.122 | 36.332 | 0.023 |
| 2031 | 0.933 | 0.201 | 0.057 | 42.709 | 0.011 |
| 2050 | 0.994 | 0.099 | 0.06 | 43.61 | 0.004 |
| Emission Factors for 3 Wheelers |  |  |  |  |  |
|  | gm/km |  |  |  |  |
|  | CO | HC | NOx | CO2 | PM |
| 2005 | 4.468 | 1.889 | 0.558 | 77.170 | 0.203 |
| 2021 | 1.55 | 0.65 | 0.385 | 85.51 | 0.025 |
| 2031 | 0.487 | 0.316 | 0.147 | 81.74 | 0.017 |
| 2050 | 0.302 | 0.206 | 0.084 | 67.53 | 0.011 |
| Emission Factors for Buses |  |  |  |  |  |
|  | gm/km |  |  |  |  |
|  | CO | HC | NOx | CO2 | PM |
| 2005 | 7.997 | 1.601 | 11.955 | 789.182 | 1.38 |
| 2021 | 2.953 | 0.295 | 4.643 | 611.479 | 0.085 |
| 2031 | 1.783 | 0.461 | 0.825 | 611.151 | 0.017 |
| 2050 | 1.599 | 0.492 | 0.649 | 611.151 | 0.011 |

| Emission Factors for Electric Cars/ Taxi |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | gm/km |  |  |  |  |
|  | CO | HC | NOx | CO2 | PM |
| 2005 | 0.000 | 0.000 | 0.001 | 0.267 | 0.000 |
| 2031 | 0.000 | 0.000 | 0.001 | 0.134 | 0.000 |
| 2050 | 0.000 | 0.000 | 0.000 | 0.080 | 0.000 |
| Emission Factors for Electric Two Wheelers |  |  |  |  |  |
|  | gm/km |  |  |  |  |
|  | CO | HC | NOx | CO2 | PM |
| 2005 | 0.000 | 0.000 | 0.000 | 0.044 | 0.000 |
| 2031 | 0.000 | 0.000 | 0.000 | 0.022 | 0.000 |
| 2050 | 0.000 | 0.000 | 0.000 | 0.013 | 0.000 |
| Emission Factors for Electric 3 Wheelers |  |  |  |  |  |
|  | gm/km |  |  |  |  |
|  | CO | HC | NOx | CO2 | PM |
| 2005 | 0.000 | 0.000 | 0.000 | 0.081 | 0.000 |
| 2031 | 0.000 | 0.000 | 0.000 | 0.041 | 0.000 |
| 2050 | 0.000 | 0.000 | 0.000 | 0.024 | 0.000 |
| Emission Factors for Electric Buses |  |  |  |  |  |
|  | gm/km |  |  |  |  |
|  | CO | HC | NOx | CO2 | PM |
| 2005 | 0.001 | 0.000 | 0.008 | 2.003 | 0.000 |
| 2031 | 0.001 | 0.000 | 0.004 | 1.002 | 0.000 |
| 2050 | 0.000 | 0.000 | 0.002 | 0.601 | 0.000 |
| Emission Factors for Electric Train |  |  |  |  |  |
|  | gm/km |  |  |  |  |
|  | CO | HC | NOx | CO2 | PM |
| 2005 | 0.009 | 0.000 | 0.052 | 13.220 | 0.002 |
| 2031 | 0.005 | 0.000 | 0.026 | 6.610 | 0.001 |
| 2050 | 0.003 | 0.000 | 0.016 | 3.966 | 0.001 |
| Emission Factors for Electric Metro/ Monorail |  |  |  |  |  |
|  | gm/km |  |  |  |  |
|  | CO | HC | NOx | CO2 | PM |
| 2005 | 0.022 | 0.000 | 0.124 | 31.328 | 0.005 |
| 2031 | 0.011 | 0.000 | 0.062 | 15.664 | 0.003 |
| 2050 | 0.007 | 0.000 | 0.037 | 9.399 | 0.002 |

Source  : Dr Munish Chandel,  IIT Mumbai (2017)